# Regeneration of a Pharmaceutical Waste Solvent by Batch Extractive Distillation

Camille Cardoze

2026

# Abstract

This Final Project investigates the regeneration of a pharmaceutical waste solvent by batch extractive distillation, focusing on the recovery of high-purity isopropyl alcohol (IPA) from an IPA–water mixture using dimethyl sulfoxide (DMSO) as entrainer, due to the fact that IPA and water form a minimum-boiling azeotrope, which prevents effective recovery by conventional batch distillation. ChemCAD simulations were used to study the influence of operating parameters, charge composition, entrainer addition, reflux ratio, feed conditions, and number of stages on product purity, recovery, and process duration.

The selected operating conditions allowed the process to exceed the required IPA purity specification of 99.75 mol%, achieving a maximum IPA distillate purity of 99.856 mol% during the main distillation step. The energy demand of the base process was evaluated, and heat-recovery alternatives were studied to reduce operating cost. Two main heat recovery strategies were compared: a pre-heater using column top vapour and a heat pump configuration based on vapour compression. Although the pre-heater required lower additional capital investment, the heat pump method provided significantly greater long-term energy savings and a shorter estimated payback period.

Material recovery strategies were also investigated by recycling off-cut streams and recovering DMSO-rich residues. These methods were assessed for their ability to reduce solvent loss, decrease waste generation, and maintain the required IPA purity. In addition, preliminary equipment sizing was carried out for the packed distillation column, storage tanks, condenser, reboiler, vacuum pump, and heat-recovery equipment. A cost estimation was then prepared for the base distillation system and the proposed heat-recovery alternatives.

Overall, the results show that batch extractive distillation with DMSO is a technically feasible method for regenerating high-purity IPA from pharmaceutical waste solvent streams. The proposed process combines solvent recovery, material recycling, and heat integration, making it a promising approach for reducing waste and improving the economic and environmental performance of solvent regeneration.

# Nomenclature

## Latin Symbols

| Symbol | Meaning | Unit |
| --- | --- | --- |
| A | Area | m² |
| Acalc | Calculated heat-transfer area | m² |
| Acond | Condenser heat-transfer area | m² |
| Atube | Tube-side flow area | m² |
| a | Bundle-diameter correlation constant | - |
| b | Bundle-diameter correlation exponent | - |
| B | Batch number | - |
| Bij | Binary interaction parameter from component i to component j | cal/mol |
| Bji | Binary interaction parameter from component j to component i | cal/mol |
| C | Cost | $ |
| C1 | Compressor 1 | MJ/h |
| C2 | Compressor 2 | MJ/h |
| CA | Corrosion allowance | mm or m |
| Cshell | Shell clearance | m |
| Ci | Installed cost | $ |
| Cp | Purchase cost | $ |
| Ctotal | Total installed cost | $ |
| Cadd | Additional installed cost | $ |
| cp | Specific heat capacity | J/(kg·K) |
| D | Diameter | m |
| Dbundle | Tube bundle diameter | m |
| E | Weld joint efficiency / Young’s modulus | - / Pa |
| Erecovery | Energy required for the recovery distillation step | GJ |
| F | F-factor / LMTD correction factor | Pa / - |
| Fc | Cost correction factor | - |
| Ft | Compressor correction factor | - |
| H | Height | m |
| HETP | Height equivalent to one theoretical plate | m |
| HP | Horsepower | hp |
| HP1 | Horsepower of compressor 1 | hp |
| HP2 | Horsepower of compressor 2 | hp |
| I | First component in binary interaction table | - |
| J | Second component in binary interaction table | - |
| k | Overall heat-transfer coefficient | W/(m²·K) |
| Kvap | Heat-transfer coefficient for vapour | W/(m²·K) |
| Kdist | Heat-transfer coefficient for distillate | W/(m²·K) |
| Kcalc | Calculated heat-transfer coefficient | W/(m²·K) |
| L | Length | m |
| Lshell | Shell length | m |
| m | Mass | kg |
| mTotal | Total mass | kg |
| m | Mass flow rate | kg/s or kg/h |
| MS | Marshall and Swift index | - |
| N | Number of stages | - |
| Nt | Number of tubes | - |
| Ncycles | Number of operating cycles | - |
| n | Amount of substance | kmol |
| nIPA | Amount of IPA | kmol |
| nH2O | Amount of water | kmol |
| Nu | Nusselt number | - |
| Nuw | Water-side Nusselt number | - |
| P | Pressure | Pa, bar, or mbar |
| P1 | Initial pressure | bar abs |
| P2 | Final pressure | bar abs |
| p | Pitch | m |
| pt | Tube pitch | m |
| Pr | Prandtl number | - |
| Prw | Water-side Prandtl number | - |
| Q | Heat duty | W or MJ/h |
| Qmax | Maximum heat duty | W or MJ/h |
| Qreb | Reboiler heat duty | W or MJ/h |
| Qfeed | Feed-heating energy | W or MJ/h |
| Qtotal | Total energy demand | GJ |
| R | Reflux ratio | - |
| Rf | Fouling resistance | m²·K/W |
| Re | Reynolds number | - |
| r | Latent heat of vaporization | J/kg |
| S | Allowable stress / required pumping speed | Pa / m³/s |
| T | Temperature | K or °C |
| T | Average temperature | K or °C |
| T1,1 | Hot-stream inlet temperature | K or °C |
| T1,2 | Hot-stream outlet temperature | K or °C |
| T2,1 | Cold-stream inlet temperature | K or °C |
| T2,2 | Cold-stream outlet temperature | K or °C |
| Tcomp. | Compressor outlet temperature | °C |
| THX1 | Temperature after heat exchanger 1 | °C |
| t | Time | h or s |
| V | Volume | m³ |
| V | Vapour volumetric flow rate | m³/s |
| Vmax | Maximum required volume | m³ |
| Vtank | Selected tank volume | m³ |
| Vpacking | Packing volume | m³ |
| VF | Vapour fraction | - |
| VFcomp. | Vapour fraction after compression | - |
| VFHX1 | Vapour fraction after heat exchanger 1 | - |
| v | Velocity | m/s |
| x | Liquid mole fraction / wall thickness | - / m or mm |
| x1 | First known independent-variable value | - |
| x2 | Second known independent-variable value | - |
| xIPA | IPA mole fraction | - |
| xH2O | Water mole fraction | - |
| xDMSO | DMSO mole fraction | - |
| xtube | Tube wall thickness | m |
| xshell | Shell wall thickness | m |
| y | Vapour mole fraction | - |
| y1 | First known dependent-variable value | - |
| y2 | Second known dependent-variable value | - |

## Greek Symbols

| Symbol | Meaning | Unit |
| --- | --- | --- |
| α | Binary interaction parameter in NRTL table | Cal/mol |
| αcond | Condensation-side heat-transfer coefficient | W/(m²·K) |
| αw | Water-side heat-transfer coefficient | W/(m²·K) |
| αboiling | Boiling-side heat-transfer coefficient | W/(m²·K) |
| αshell | Shell-side heat-transfer coefficient | W/(m²·K) |
| αst | Steam condensation coefficient | W/(m²·K) |
| ΔP | Pressure jump or pressure drop | Pa or mbar |
| ΔPtotal | Total pressure jump across packed section | mbar |
| ΔT | Temperature difference | K or °C |
| ΔT1 | First terminal temperature difference | K |
| ΔT2 | Second terminal temperature difference | K |
| ΔTlm | Logarithmic mean temperature difference | K |
| ΔTm | Mean temperature difference | K |
| ΔTlog | Logarithmic mean temperature difference | K |
| ΔC | Difference in capital cost | $ |
| λ | Thermal conductivity | W/(m·K) |
| μ | Dynamic viscosity | Pa·s |
| ν | Poisson’s ratio | - |
| π | Mathematical constant pi | - |
| ρ | Density | kg/m³ |
| σ | Surface tension | N/m |
| ∞ | Infinite reflux condition | - |

## Subscripts, indices and Conventions

| Symbol / convention | Meaning |
| --- | --- |
| i | First component index |
| j | Second component index |
| 1 | First point, side, or terminal value |
| 2 | Second point, side, or terminal value |
| 1,1 | Hot-stream inlet |
| 1,2 | Hot-stream outlet |
| 2,1 | Cold-stream inlet |
| 2,2 | Cold-stream outlet |
| in | Inlet or inside basis |
| out | Outlet or outside basis |
| avg | Average value |
| real | Real or estimated value |
| calc | Calculated value |
| clean | Clean condition without fouling |
| dirty | Dirty condition including fouling |
| cond | Condensation or condenser side |
| boiling | Boiling side |
| shell | Shell side or shell geometry |
| tube | Tube side or tube geometry |
| wall | Heat-transfer wall |
| film | Film condition |
| w | Water or cooling water |
| st | Steam |
| l | Liquid phase |
| v | Vapour phase |
| IPA | Isopropyl alcohol or isopropanol |
| H₂O | Water |
| DMSO | Dimethyl sulfoxide |
| RO | Run-off stream |
| Chg. | Charge |
| Dist. | Distillate |
| L | Liquid stream |
| V | Vapour stream |
| top | Column top |
| bottom | Column bottom |
| external | External pressure condition |
| cr | Critical value |
| max | Maximum value |
| final | Final selected value |
| lm | Logarithmic mean |
| log | Logarithmic mean |
| m | Mean or corrected mean |
| reb | Reboiler |
| main | Main IPA distillation step |
| feed | Relating to the feed |
| process | Complete process |
| total | Total value |
| recovery | Recovery step |
| purchase | Purchase cost |
| random | Random packing |
| unit | Unit cost |
| packing | Packing |
| base | Base distillation system |
| pre | Pre-heater |
| HP | Heat-pump method |
| add | Additional cost |
| HX | Heat exchanger |
| HPS | High-pressure steam |
| comp | Compressor |
| int | Inter-cooler |
| tank | Tank |
| col | Column |
| bundle | Tube bundle |
| ( ) | Flow rate or rate with respect to time |
| ( ) | Average value |
| [-] | Dimensionless value |
| ≤ | Less than or equal to |
| ≥ | Greater than or equal to |
| AEL | TEMA exchanger type code |
| BKU | TEMA exchanger type code |

## Abbreviations

| Abbreviation | Meaning |
| --- | --- |
| IPA | Isopropyl alcohol / isopropanol |
| DMSO | Dimethyl sulfoxide |
| H₂O | Water |
| NRTL | Non-random two-liquid model |
| BIP | Binary interaction parameter(s) |
| VLE | Vapour-liquid equilibrium |
| HETP | Height equivalent to one theoretical plate |
| LMTD | Logarithmic mean temperature difference |
| RO | Run-off |
| Chg. | Charge |
| Dist. | Distillate |
| HX | Heat exchanger |
| HX1 | Heat exchanger 1 |
| HX2 | Heat exchanger 2 |
| IC | Inter-cooler |
| VF | Vapour fraction |
| HP | Horsepower |
| HPS | High-pressure steam |
| MS | Marshall and Swift index |
| TEMA | Tubular Exchanger Manufacturers Association |
| MJ | Megajoule |
| GJ | Gigajoule |
| kW | Kilowatt |
| W | Watt |
| Pa | Pascal |
| mbar | Millibar |
| bar abs | Absolute bar pressure |
| mol% | Mole percent |
| mass% | Mass percent |
| kmol | Kilomole |
| kg/h | Kilogram per hour |
| m³/h | Cubic meter per hour |
| W/m²K | Watt per square meter kelvin |
| m²K/W | Square meter kelvin per watt |
| Cal/mol | Calorie per mole |
| $/GJ | Dollars per gigajoule |
| €/m³ | Euros per cubic meter |

# Introduction

The main goal of this research project is to investigate a batch extractive distillation process using dimethyl sulfoxide (DMSO) as a solvent to recover a distillate containing at least 99.75 mol% isopropyl alcohol (IPA). This separation is especially relevant to the pharmaceutical industry, where IPA is commonly used and is often required in very high concentrations. The mixture to be separated contains water, IPA, and a negligible amount of toluene. Dimethyl sulfoxide will be used as feed. Batch distillation cannot be used effectively for this regeneration process, since water and IPA form a homogeneous minimum-boiling azeotrope at approximately 87–88 mass% IPA and 80.2–80.4 °C (Nemeth et al. 2019).

Furthermore, the project aims to analyze how operational and geometrical parameters affect the performance of the distillation process and the column dimensions required for efficient operation. In addition, the study also examines different strategies for improving solvent and material recovery, reducing waste, and increasing the energetic efficiency of the process.

Extractive distillation involves the continuous addition of a usually low-volatility extra component to a mixture when separation by conventional distillation is not possible. This additional component increases the relative volatility difference between the components to be separated, thereby allowing the azeotrope to be bypassed. It differs from azeotropic distillation, in which the third component, often referred to as an entrainer or extractive agent, forms a new low-boiling azeotrope with one of the original components and is recovered from the distillate rather than from the bottom of the column (Gerbaud et al. 2019).

DMSO was selected as the solvent for this process because of its very high boiling point, which is significantly greater than that of both water and IPA. It is a stable, cost-effective, and non-hazardous solvent with a strong affinity for water, while also being easily recyclable; most importantly, it has a favorable effect on the relative volume. These properties make it suitable for extractive distillation processes where high-purity IPA recovery is required (Nemeth et al. 2019).

For this project, ChemCAD modeling software was used to simulate the batch extractive distillation process, predict phase compositions and mixture behavior, obtain numerical data, and optimize the operating conditions. Modeling is essential because it allows the separation process to be evaluated and improved without the need to perform numerous time-consuming laboratory experiments. Several ChemCAD simulations were carried out to study not only the original separation process, but also different recycling and energy-saving configurations.

Further simulations were conducted to investigate the possibility of recycling off-cut material. In one approach, off-cut material from a previous batch was added directly to the new charge, while the remaining DMSO recovered from the previous simulation was reused as the entrainer. In another approach, the IPA-water off-cut material was mixed with the DMSO-water off-cut product from the previous distillation, and the resulting mixture was added to the new charge. These simulations were performed to evaluate whether valuable material could be recovered and reused while maintaining the required IPA purity in the distillate.

Energy-saving alternatives were also investigated. One simulation considered preheating the feed using vapor from the top of the distillation column, with the aim of reducing the energy required to heat the feed before separation. A further dynamic simulation was performed in which the top vapor was compressed and its heat was used in the reboiler to heat the charge. This configuration was studied as a potential heat-integration strategy to decrease the overall energy demand of the batch distillation operation.

In addition to the process simulations, the two main heat exchangers of the system, namely the condenser and the reboiler, were designed. Their main dimensions, number of tubes, wall thicknesses, and suitable exchanger types were determined. Finally, a cost estimation was carried out for the complete distillation column, including the main equipment required for operation. This provides a broader technical and economic evaluation of the proposed separation process.

Among the benefits of this distillation strategy are its versatility, reduced waste generation, improved solvent recovery, scalability, and potentially lower cost compared with alternative separation methods. Since DMSO can be applied in a wide range of pharmaceutical processes and enables the recovery of high-purity IPA from azeotropic mixtures, the results of this study may provide a sustainable and industrially relevant solution for handling IPA-water mixtures.

# Theoretical Background

## Batch Extractive Distillation

In batch extractive distillation, a mixture made up of a more volatile and a less volatile component is heated up to achieve the component separation, by causing the more volatile component to evaporate in a higher concentration and then condensate for its recovery. This process is done in batch, where only a portion of the mixture is treated in a single cycle; a fixed amount of it is to be separated at a time (Budapest University of Technology and Economics, Department of Building Service Engineering and Process Engineering 2023).

The distillate of the mixture is separated from the less volatile substances that are left in the residue or at the bottom of the tanks, by being brought back to its liquid phase by a condenser and then collected. However, part of this condensate will return to the column as reflux to improve the mass transfer between phases and therefore the efficiency of the column, at a ratio called reflux ratio (R). It is the quotient of the liquid used as reflux and the total liquid withdrawn (distillate). Under constant reflux ratio, the composition of the distillate will steadily change (the more volatile component’s mole fraction will decrease in the distillate) as the procedure progresses.

## Azeotropes

An azeotrope is a specific mixture composition where the liquid and vapour phases in vapour–liquid equilibrium have the same composition. In ordinary distillation, the vapour phase is usually richer in the more volatile component than the liquid phase, which allows separation to occur. At the azeotropic composition, however, the vapour produced during boiling has the same composition as the liquid, meaning that no further enrichment of one component is achieved by simple vaporization and condensation. As a result, conventional distillation cannot separate the mixture beyond the azeotropic composition without changing the operating conditions or introducing an additional separating agent.

Both phases meet on the main diagonal line of the equilibrium curve diagram; this would mean that x = y. This condition prevents any more separation from happening, since the equilibrium reached eliminates the gradient needed for further change in the concentrations (Budapest University of Technology and Economics, Department of Building Service Engineering and Process Engineering 2023). In the case, the azeotrope presents at a mass fraction of 87-88% and a temperature of 80.2 – 80.4 °C. This is due to the interactions between the molecules (see Table 3.1) of each component in the solution.

## Material Recovery in Extractive Distillation

Material recovery is an important part of extractive distillation, since the process requires the addition of a third component. The purpose of this component is to change the relative volatility of the original components and make the separation possible. However, because the solvent is not the desired final product, it should be recovered and reused whenever possible. This reduces the amount of fresh solvent required, lowers operating costs, and decreases waste formation. In extractive distillation, the solvent is usually chosen to have a much higher boiling point than the components being separated, so it is mostly recovered from the bottom product rather than from the distillate. (Seader et al. 2011).

In batch extractive distillation, the recovery of materials is especially relevant because the composition of the liquid in the still pot changes continuously with time. As the operation progresses, different fractions may be obtained: some fractions may meet the desired purity specification, while others may contain valuable amounts of IPA, water, or DMSO but not be pure enough to be considered final products. These intermediate or run-off streams can often be recycled to a later batch or treated in a separate recovery step.

For this project, material recovery is studied through the recycling of run-off material and the recovery of DMSO from previous distillation residues. Since DMSO acts as the extractive solvent, its recovery is especially necessary for improving the economic and environmental performance of the process.

## Heat Exchangers

Heat exchangers are devices used to transfer heat between two fluids at different temperatures. In most industrial heat exchangers, the fluids do not mix directly, but are separated by a solid wall through which heat is transferred. The heat transfer process usually involves convection from the hot fluid to the wall, conduction through the wall, and convection from the wall to the cold fluid. Heat exchangers are widely used in chemical and mechanical engineering for heating, cooling, condensation, evaporation, and heat recovery (Kern 1950). The most common configuration in the case of condensers and boilers is the shell and tube, single-pass heat exchanger; see figure 2.8 for an illustration.

In distillation systems, the two most important heat exchangers are the condenser and the reboiler. The condenser removes heat from the vapor leaving the top of the column and converts it into liquid. Part of this liquid may be withdrawn as distillate, while the remaining part is returned to the column as reflux. The reboiler, on the other hand, supplies heat to the liquid at the bottom of the column, producing vapor that rises through the column and allows mass transfer between vapor and liquid phases.

The design of a heat exchanger depends mainly on the required heat duty, the temperature difference between the fluids, the heat transfer coefficient, and the available heat transfer area. A common design equation is:

Q=KAFΔTlm  (1)

where Q is the heat duty, K is the overall heat transfer coefficient, A is the heat transfer area, ΔTlm is the logarithmic mean temperature difference, and F is a correction factor. In this project, this theory is applied to the design of the distillation column’s condenser and reboiler.

## Heat Recycling in the Industry

Heat recycling, also called heat recovery, is the reuse of heat that would otherwise be rejected from a process. In many industrial systems, some streams must be cooled while others must be heated. Instead of using only external utilities, such as cooling water and steam, heat can be transferred from hot process streams to cold process streams. This reduces the energy demand of the plant and improves overall process efficiency. (Kemp 2007).

In distillation, heat recycling is especially important because the process usually requires significant energy input in the reboiler and significant heat removal in the condenser. One simple form of heat recycling is feed preheating, where heat from a hot stream, such as overhead vapor or bottom product, is used to increase the temperature of the feed before it enters the column. This decreases the amount of heat that must later be supplied by the reboiler.

A more advanced method is vapor recompression. In this method, the vapor leaving the top of the column is compressed, which increases its temperature and pressure. The compressed vapor can then be used as a heating medium in the reboiler. Although the compressor requires mechanical work, part of the heat that would normally be removed in the condenser is reused. The goal is for the net output thermal energy of the recycling system to be higher than the required input energy.

# Mixture Composition

## Initial Conditions

Our mixture to be separated has a volume of 3500 liters, and a composition of 66.6 mol% isopropanol (IPA), and 33.4 mol% water. The time available to complete the distillation process is 22 hours. Inside the batch distillation column, the temperature will gradually increase until it reaches the boiling point of the mixture, 80.2 °C. Our column was assigned a number of theoretical stages of n = 12 (not including the total condenser and the still pot). The feed stage initially is the 5th, counted from the top. As for the entrainer, dimethyl sulfoxide (DMSO) will be fed to the column at 100% purity, room temperature (25 °C) and 1.013 bar in pressure. The chosen starting feed flow rate is 5 kmol/h. These parameters are to be changed depending on the results.

Binary interaction parameters.

| I | J | Bij [Cal/mol] | Bji [cal/mol] | Alpha [cal/mol] |
| --- | --- | --- | --- | --- |
| Isopropanol | Water | 20.0554 | 832.981 | 0.3255 |
| Isopropanol | Dimethyl Sulfoxide | 115.279 | -25.0123 | 0.4 |
| Water | Dimethyl Sulfoxide | 1203.77 | -524.822 | 0.6615 |

In Table 3.1 can be found the binary interaction parameters (BIPs) for the mixture, which are the coefficients to be used in the NRTL thermodynamic model to describe the interactions between pairs of molecules in the mixture. The BIPs describe the excess Gibbs free energy of mixing (Yao et al. 2007).

The process will start with total reflux (R = ∞) and after the steady-state is reached, it will be working with an initial reflux ratio of R = 6, subject to change as the analysis progresses. In the same manner, It will start with an initial feed temperature of 20°C and 12 theoretical stages for the column.

## Residue Curves

Residue curves (Figure 3.2) depict how the liquid composition in the distillation still-pot evolves over time during simple batch distillation at 1.01 bar. Each vertex of the triangle represents a pure component, and for the IPA-water-DMSO system the curves start at the azeotrope, showing the need for DMSO to carry out the separation. They, too, illustrate the separation trajectory: the concentration of DMSO increases as time progresses for every case, revealing that the distillation of this mixture will generally lead to DMSO accumulation in the pot, while IPA and water are removed. Similarly, the diagram tells us that DMSO forms no azeotropes with the other two components.

## Effect of DMSO on the Azeotrope

The altered activity coefficients increase the relative volatility of IPA compared to water. This shifts the VLE curve, eliminating the original azeotropic point where x = y. The high boiling point of DMSO (189°C) ensures it mostly remains in the liquid phase, accumulating in the column’s extractive section. This prevents IPA-water azeotrope reformation and allows IPA to distill overhead. As it can be seen in Figure 3.3, The IPA-water-DMSO system no longer exhibits an azeotrope, enabling distillation to achieve >99.75% IPA purity.

The VLE diagrams show that the addition of DMSO to a solvent-free concentration changes the phase-equilibrium behavior of the IPA-water system. Without an entrainer, the IPA-water mixture forms an azeotrope, which limits the maximum IPA purity attainable by ordinary distillation. When DMSO is added, the equilibrium curve is shifted away from the diagonal, meaning that the vapour and liquid compositions are no longer equal at the original azeotropic point. This confirms that DMSO successfully changes the relative volatility of IPA and water, allowing IPA to be concentrated beyond the azeotropic composition. The effect becomes more visible as the DMSO mole fraction increases, showing that the entrainer is essential for achieving the required high-purity IPA distillate.

# Parameter Effects

## Targets

in this chapter we will only study the IPA recovery step, or the dehydration of isopropanol. The distillate’s solution must be ≥ 99.75 mol% IPA, ≤ 0.25 mol% water, and ≤ 0.15 mol% dimethyl sulfoxide (DMSO). In the water recovery step, the water will be withdrawn as distillate. In the water obtained, less than 0.05 mol% IPA should be present, and less than 0.035 mol% DMSO. After the whole process is finished, the remaining DMSO from the bottom will be collected from the column; The following purity requirements will be used: ≥ 99.75 mol% DMSO, ≤ 0.25 mol% water, and ≤ 0.15 mol% IPA.

## Parameters studied

- Feed stage [-]

- Reflux ratio [-]

- Number of stages [-]

- Feed flow rate [kmol/h]

- Feed Temperature [°C]

Dependent variables:

- Mass of the IPA product [kg]

- Time [h]

- Molar fraction (purity) of the product [-]

Goals:

- Maximum mass

- Minimum time

- Meeting purity requirements

Constant values:

- Volume of charge: 3500 L

- Charge temperature: 80°C

- Reboiler duty: 1000 MJ/h

- Charge composition:

  - 0.66 mole fraction IPA

  - 0.34 mole fraction water

- Feed composition: 1 mole fraction DMSO

We have adhered to an interval of 15-30 for the number of stages, 5-10 kmol/h for the feed rate, 4-8 for the feed stage, 50-70 °C for the feed temperature, and 4-6 for the reflux ratio, as these are common industrial values that have proved to have a good efficiency of separation in literature (Nemeth et al. 2019), (Gerbaud et al. 2019). A quite difficult, azeotropic separation calls for several stages and a high reflux ratio.

The stopping criteria selected was a purity of 0.9998 mol fraction IPA in the accumulator, trending downwards; this ensures that once the purity of IPA in the distillate contained in the accumulator tank starts to fall below 98 mol%, the process will terminate.

The IPA purity results and total elapsed time of the last successful simulation attempts tried with different values for the feed stage, reflux ratio, feed rate, stage number and feed temperature are shown in Tables 4.1, 4.2, 4.3, 4.4 and 4.5, respectively. Data was obtained from sensitivity analyses. The final parameters were chosen through an iterative process, in which a sensitivity analysis is conducted, parameters were changed to achieve optimal results, and another sensitivity analysis is carried out until results are satisfactory.

Feed Stage Data

| Trial | Feed Stage | Mole Fraction of IPA | Time (h) | Mass (kg) |
| --- | --- | --- | --- | --- |
| 1 | 1 | 0.727319 | 8.45001 | 3509.90 |
| 2 | 3 | 0.997528 | 9.20002 | 2380.31 |
| 3 | 6 | 0.997400 | 9.20002 | 2381.18 |
| 4 | 9 | 0.997040 | 9.20002 | 2380.57 |
| 5 | 11 | 0.997273 | 9.15002 | 2370.28 |
| 6 | 14 | 0.995347 | 8.50001 | 2225.38 |
| 7 | 17 | 0.986126 | 8.15001 | 2132.24 |
| 8 | 19 | 0.970703 | 7.90001 | 2051.91 |
| 9 | 22 | 0.904141 | 8.20001 | 2021.87 |
| 10 | 25 | 0.717659 | 13.55 | 2696.94 |

Which can be plotted as:

Reflux Ratio Data

| Trial | Reflux Ratio | Mole Fraction of IPA | Time (h) | Mass (kg) |
| --- | --- | --- | --- | --- |
| 1 | 3 | 0.996872 | 16.15 | 2393.41 |
| 2 | 4 | 0.997535 | 18.05 | 2406.52 |
| 3 | 5 | 0.997973 | 19.95 | 2413.92 |
| 4 | 6 | 0.997836 | 21.90 | 2424.64 |
| 5 | 7 | 0.997399 | 23.7999 | 2426.82 |

Which can be plotted as:

Feed Rate Data

| Trial | Feed Rate (kmol/h) | Mole Fraction of IPA | Time (h) | Mass (kg) |
| --- | --- | --- | --- | --- |
| 1 | 5 | 0.987313 | 8.60001 | 2375.32 |
| 2 | 6.11111 | 0.995814 | 8.65001 | 2356.56 |
| 3 | 7.22222 | 0.997105 | 8.95002 | 2381.33 |
| 4 | 8.33334 | 0.996503 | 9.20002 | 2387.23 |
| 5 | 9.44445 | 0.997112 | 9.40002 | 2380.46 |
| 6 | 10.5556 | 0.996768 | 9.65002 | 2379.43 |
| 7 | 11.6667 | 0.996776 | 9.90002 | 2375.50 |
| 8 | 12.7778 | 0.997171 | 10.15 | 2368.91 |
| 9 | 13.8889 | 0.996996 | 10.45 | 2367.03 |
| 10 | 15 | 0.997244 | 10.75 | 2362.07 |

Which can be plotted as:

Stage Number Data

| Trial | Stage Number | Mole Fraction of IPA | Time (h) | Mass (kg) |
| --- | --- | --- | --- | --- |
| 1 | 10 | 0.907955 | 8.15001 | 2016.42 |
| 2 | 12.2222 | 0.958189 | 7.85001 | 2022.11 |
| 3 | 14.4444 | 0.980373 | 8.05001 | 2100.75 |
| 4 | 16.6667 | 0.990602 | 8.30001 | 2173.13 |
| 5 | 18.8889 | 0.995448 | 8.50001 | 2225.54 |
| 6 | 21.1111 | 0.997299 | 9.15002 | 2370.32 |
| 7 | 23.3333 | 0.997048 | 9.20002 | 2380.59 |
| 8 | 25.5556 | 0.997349 | 9.20002 | 2381.05 |
| 9 | 27.7778 | 0.997496 | 9.20002 | 2381.28 |
| 10 | 30 | 0.997590 | 9.20002 | 2381.42 |

Which can be plotted as:

Feed Temperature Data

| Trial | Feed Temperature (°C) | Mole Fraction of IPA | Time (h) | Mass (kg) |
| --- | --- | --- | --- | --- |
| 1 | 20 | 0.996916 | 9.90002 | 2391.57 |
| 2 | 32 | 0.996842 | 9.70002 | 2390.15 |
| 3 | 44 | 0.996943 | 9.50002 | 2387.46 |
| 4 | 56 | 0.997194 | 9.30002 | 2383.51 |
| 5 | 68 | 0.997534 | 9.10002 | 2378.27 |
| 6 | 80 | 0.996955 | 8.95002 | 2381.23 |

Which can be plotted as:

## Conclusion on Parameter Effects

The sensitivity analyses show that each parameter affects the IPA distillate purity, recovered mass, and elapsed time through different mechanisms: The feed stage strongly influences the contact between the DMSO feed and the internal vapour-liquid flows (Figure 4.1). If the feed is introduced in too high of a stage, it starts appearing in the distillate; if it is introduced too low, it cannot modify the relative volatility across enough of the column. For this reason, adequate results occur at intermediate feed stages, while the extremes show clear drops in purity and, in one case, a strong increase in time. The reflux ratio improves separation by returning more condensed liquid to the column, increasing vapour-liquid contact. However, after a certain point, higher reflux gives only small purity improvements while greatly increasing the operating time. The purity peak at the middle reflux values is an indicator of the fittest option (Figure 4.2). The feed flow rate mainly controls how much DMSO is available to break the IPA-water azeotropic behavior. Too low a flow rate gives insufficient solvent action, while too high a flow rate increases the elapsed time and can slightly reduce the recovered mass (Figure 4.3); therefore, an intermediate feed rate is preferred. Increasing the number of stages improves purity and mass because more equilibrium contacts are available, but the effect reaches a plateau at higher stage numbers, meaning that additional stages give diminishing returns and would mainly increase column size and cost (Figure 4.4). Finally, increasing feed temperature reduces the elapsed time because the feed enters closer to column conditions, but excessive heating can disturb the internal temperature and composition profiles (Figure 4.5). The purity peak occurs at a moderately high feed temperature, while the recovered mass tends to decrease as temperature rises. Overall, the selected parameter values should represent a compromise between high purity, high recovered mass, and low elapsed time, rather than the maximum value of only one dependent variable.

Selected parameter values.

| Feed Stage | Reflux Ratio | Feed Flow Rate | Number of Stages | Feed Temperature |
| --- | --- | --- | --- | --- |
| 4 | 4 | 7.5 kmol/h | 24 | 62 °C |

These final parameters displayed in Table 4.6 were chosen taking maximum efficiency into consideration; they produce balanced results regarding purity, elapsed time and mass. We selected 24 stages for the column because, even though the elapsed time is high, it will result in sufficient purity and a great amount of mass. A feed stage of 4 gives us a good balance between distillate purity and mass, while keeping time at a minimum. As for the selected feed flow rate, a value between 7 and 8 kmol/h will be suitable, since it was possible to complete the process at a relatively short amount of time and obtain a satisfactory amount of distillate mass, with high purity. Therefore, 7.5 kmol/h is acceptable, or 586 kg/h. A reflux ratio of 6 would give us the best results according to the graph; however, the time and the energy it needs are significantly greater. A reflux ratio of 4 gives sufficient results, while making up for time. Feed temperature is chosen to be 62°C because, even though it will not produce the highest amount of distillate possible, it will reduce the elapsed time and produce a high IPA purity.

# Distillation Results

A maximum purity of 99.856 mol% IPA was achieved, meeting the requirements; 0.14 ≤ 0.25 mol% water, 0.0012 ≤ 0.15 mol% DMSO; these results calculated by ChemCAD are shown in Figure 5.1. The total elapsed time was 9.05 hours.

In Figure 5.2, it is possible to observe how the composition of the distillate changed with time, under the chosen parameter values. This plot was produced by the ChemCAD simulation.

The distillate composition graph shows that the IPA mole fraction remains very high during most of the main distillation step, while the water and DMSO mole fractions remain low. This confirms that the selected operating parameters allow IPA to be removed as the main overhead product with only small amounts of impurities. Near the end of the step, the IPA mole fraction begins to decrease while the impurity content increases, indicating that the useful IPA-rich distillation period is ending. Therefore, the stopping point was selected before a significant decrease in distillate purity occurs, in order to maintain the required product specification.

## Additional Steps

After obtaining the desired product, the distillation column will still have liquid at the bottom, whose composition is still rich in all three components: water, DMSO and IPA.

In an attempt to recover as much water as possible, as well as solvent, three more steps to the process were added: a water-IPA run-off step, a water distillation step, and a water-DMSO run-off step. The new simulation layout, after adding the necessary equipment for these additional steps, is illustrated in Figure 5.3.

In the water-IPA run-off step, the goal is to dispose of all leftover IPA in the bottom, in order to produce water more efficiently in the next step. In the water distillation step, as much water as possible will be separated from the leftover mixture (less than 0.05 mol% IPA should be present, and less than 0.035 mol% DMSO). Finally, for the water-DMSO run-off step, it is attempted to leave as little water in the bottom as possible in order to recover the solvent.

In this case, extensive sensitivity analyses were not carried out, since the investigated parameters were now set to the chosen values for the whole process. Instead, the stop values for each step were set as the following:

- stop value for step 2: 0.8000 mole fraction water in the accumulator

- stop value for step 3: 0.9995 mole fraction water in the accumulator

- stop value for step 4: 0.9995 mole fraction DMSO in the still pot

The stop values were chosen according to the purpose of each additional distillation step. In step 2, the objective is not to obtain a final pure product, but to remove the remaining IPA-rich fraction before the main water recovery step. Therefore, the step is stopped when the accumulator reaches a high water content, indicating that most of the remaining IPA has already been removed from the bottom mixture. In step 3, the goal is to recover water as a final product, so a much stricter stop value is used: the accumulator must reach approximately pure water. In step 4, the objective is to recover the DMSO remaining in the still pot, so the process is continued until the bottom product reaches the required DMSO purity.

The reflux ratios for each step were chosen in a small iteration process, using values ranging from R = 2 to 5 (Budapest University of Technology and Economics, Department of Building Service Engineering and Process Engineering 2023). After four simulations (corresponding to the four integer values selected) were carried out for each step, the following values yielded the best results in the least possible amount of time:

- R for step 2: 5

- R for step 3: 4

- R for step 4: 2

A higher reflux ratio was used in step 2 because this step handles a transition mixture containing IPA and water, and stronger internal liquid-vapour contact is needed to remove the remaining IPA effectively. Step 3 uses a moderate reflux ratio because water is the main volatile component and can be recovered at high purity without excessive reflux. Step 4 uses the lowest reflux ratio because the target product is the DMSO-rich bottom stream rather than the distillate; therefore, very high reflux is not necessary and would only increase the operating time.

This will allow us to carry out the simulation until the desired purity for both water and DMSO are reached.

The resulting graphs of the additional distillation steps are showcased in Figures 5.4, 5.5 and 5.6.

After the process was finished, the results were:

- Water distillate:

  - 226.52 kg, 0.9994 mol fraction water, 0.0004432 mol fraction IPA, 0.0001375 mol fraction DMSO

- DMSO bottom:

  - 4060.02 kg, 0.9995 mol fraction DMSO, 0.0004904 mol fraction water, 7.226e-18 mol fraction IPA

The results are satisfying and the liquids are usable for other applications. The total elapsed time was 16.15 hours, still under the 22 hour goal. Run-off steps will later be re-integrated into the system, further explained in Chapter 8, in order to recover as much material as possible. These results support the inclusion of material recovery steps in the overall process design, as they reduce waste formation and improve the reuse potential of both water and DMSO.

# Column Geometry

Our next step is to calculate the main dimensions of the column; that is, its column diameter, tank diameter, volume, height and thickness. In order to do this, we will need to obtain the flow rate of the vapour phase and the density of both the vapour and liquid phases in every stage throughout the column. This data was obtained from the ChemCAD software, given by the tray analysis report.

## F-factor and HETP value

The F factor is a parameter that represents the momentum of the vapour phase. From the Sulzer packing catalogue, the structured MellapakPlus 252.Y packing was chosen, as it offers a high operational efficiency and minimum hold-up for atmospheric pressure distillation (Sulzer Chemtech 2024). It is widely used in the pharmaceutical and food industry. In the same catalogue, it is possible to see the HETP-F factor graphs for each packing type. The F-factor will be picked so that it will not cause flooding or weeping in the column; 2 √Pa was selected (See Figure 6.1).

In the same catalogue section, we have a graph indicating what HETP (Height Equivalent to one Theoretical Plate) value corresponds to an F factor of 2: 0.4 m (see Figure 6.2). The blue line refers to the distillation pressure 960 mbar.

## Column Height

Since the HETP value is known, the calculation of the column height can be carried out directly. The HETP represents the height of packing required to achieve the separation effect of one theoretical stage.

The column height is calculated using the following equation (Budapest University of Technology and Economics, Department of Building Service Engineering and Process Engineering 2023):

H=N×HETP  (2)

where:

- H = packed column height, in m

- N = number of theoretical stages

- HETP = height equivalent to one theoretical plate, in m

The column was chosen to have 24 theoretical stages, and the selected HETP value for the chosen packing is HETP = 0.4 m. Therefore, the required packed height is:

H=24×0.4=9.6 m  (3)

The selected column height is 9.6 m, which can be rounded up to 10 m to allow for some extra safety space. This height is only representative of the column alone, without accounting for the reboiler, bottom tank or condenser.

## Pressure Drop

The pressure drop is the decrease in pressure that occurs as the vapour phase flows through the packing of the distillation column. In a packed column, this pressure loss is mainly caused by friction between the rising vapour, the descending liquid, and the surface of the packing. It is an important design parameter because excessive pressure loss can increase the pressure and temperature at the bottom of the column, which may negatively affect the separation and increase the operating requirements of the process. For a packed distillation column, the total pressure drop can generally be expressed as (Sulzer Chemtech 2024):

ΔPtotal=ΔPH⋅H

where:

- ΔPtotal = total pressure drop across the packed section, in mbar.

- ΔPH = pressure drop per unit height of packing, in mbar/m.

- H = total packed height of the column, in m.

For a distillation column, the bottom pressure can then be estimated from the top pressure and the total column pressure drop:

Pbottom=Ptop+ΔPtotal

However, in the Sulzer catalogue’s MellapakPlus 252.Y packing section, a relationship between the pressure drop and the F-factor is already provided:

ΔPH=1 mbar/m  (4)

Since the packed height of the column is H = 10 m, the total pressure drop is:

ΔPtotal=1 mbar/m⋅10 m=10 mbar

Therefore, the pressure at the bottom of the packed section is expected to be approximately 10 mbar higher than the pressure at the top of the column. The calculated pressure drop is small compared with the operating pressure of the distillation column. Therefore, the selected MellapakPlus 252.Y structured packing is considered suitable, since it provides the required separation height while maintaining a low pressure drop. The 10 mbar pressure increase across the 10 m packed section is not expected to significantly affect the vapour-liquid equilibrium or the overall column operation.

## Column Diameter

In order to calculate the diameter of the column, the vapour phase velocity on each stage must first be evaluated by rearranging the F-value equation, whose value we selected: (Sulzer Chemtech 2024):

F=vρv  (5)

Solving for the vapour velocity:

v=Fρv  (6)

where:

- F = F-factor, in Pa

- v = maximum vapour velocity, in m/s

- ρv = vapour phase density, in kg/m3

With the vapour velocity, the cross-sectional area of the column can be calculated by rearranging the continuity equation:

A=Vv  (7)

where:

- A = cross-sectional area of the column, in m2

- V = vapour volumetric flow rate, in m3/s

- v = vapour velocity, in m/s

The column diameter is then calculated from the circular area equation:

A=πD24  (8)

Solving for the diameter:

D=4Aπ  (9)

For each step of the process, the highest vapour volumetric flow rate and the lowest vapour density were considered in order to calculate the largest required column diameter, and these were obtained from ChemCAD’s tray properties’ reports (see Table 6.1). This approach was used for safety, since the column must be able to operate under the most demanding vapour-flow conditions.

Diameter calculation results.

| Simulation Steps | Vapour vol. flow rate [m3/s] | Vapour density [kg/m3] | Vapour velocity [m/s] | Area [m^2] | Diameter [m] |
| --- | --- | --- | --- | --- | --- |
| 0 | 0.632 | 1.625 | 1.5689 | 0.314 | 0.496 |
| 1 | 0.174 | 0.853 | 2.1655 | 0.0803 | 0.320 |
| 2 | 0.210 | 0.595 | 2.5928 | 0.0810 | 0.321 |
| 3 | 0.215 | 0.593 | 2.5972 | 0.0828 | 0.325 |
| 4 | 0.240 | 2.100 | 1.3801 | 0.1739 | 0.471 |

As shown in Table 6.1, the highest calculated diameter value is 0.632 m. To provide an additional safety margin, a safety factor of 20% is applied:

D=0.632×1.20=0.758 m  (10)

Therefore, the final selected column diameter is 0.76 m after rounding up the second decimal, for the sake of simplicity.

## Column Thickness

The wall thickness of the distillation column was calculated by treating the column as a cylindrical pressure vessel under internal pressure. For safety purposes, the design pressure was selected as 6 bar, which is higher than the expected operating pressure of the column. The column is assumed to be made of carbon steel, with an allowable stress of S=120×106 Pa and a weld joint efficiency of E=0.85. (ASME 2025b)

The minimum required wall thickness was calculated using the cylindrical vessel equation (ASME 2025b):

x=PRSE−0.6P  (11)

where:

- x = minimum required wall thickness, in m

- P = design pressure, in Pa

- R = internal radius of the column, in m

- S = allowable stress of the construction material, in Pa

- E = weld joint efficiency, dimensionless

For this calculation, the following values used can be found in Table 6.2.

Parameters relevant to wall thickness.

| Parameter | Symbol | Value | Unit |
| --- | --- | --- | --- |
| Design pressure | P | 600000 | Pa |
| Column radius | R | 0.38 | m |
| Allowable stress of carbon steel | S | 120×106 | Pa |
| Weld joint efficiency | E | 0.85 | - |

Substituting the values:

x=600000×0.38(120×106)(0.85)−0.6(600000)=0.00224 m=2.44 mm  (12)

The calculated theoretical wall thickness is therefore 2.44 mm. However, this value only represents the minimum thickness required to withstand the selected internal design pressure. A corrosion allowance must also be included, especially because carbon steel is used as the construction material. Assuming a corrosion allowance of 3.2 mm (Coulson et al. 1999):

xfinal=t+CA=2.44+3.2=5.64 mm

The practical wall thickness is rounded up to 6 mm in order to guarantee safety.

The column is later planned to operate at a pressure of 0.1 bar. Since the outside of the column is assumed to be at atmospheric pressure, this condition creates an external pressure difference of approximately Pexternal=1.013−0.1=0.913 bar=91300 Pa

A preliminary external pressure check shows that a 6 mm wall thickness can withstand the vacuum condition if the full 5 mm thickness is available structurally. To check for shell buckling, the following expression was used (Hilburger 2020):

Pcr=2E3(1−ν2)xR3  (13)

where Pcr is the critical external pressure, E is Young’s modulus, ν is Poisson’s ratio, t is the wall thickness, and R is the column radius.

Using carbon steel properties (ASME 2025a):

E=200⋅109 Pa

ν=0.30

and the selected column dimensions:

x=0.006 m

R=0.38 m

the critical external pressure is:

Pcr=2(200⋅109)3(1−0.382)0.0060.383=9.82⋅105 Pa=9.82 bar  (14)

Since the estimated critical external pressure is higher than the vacuum pressure difference, the selected 6 mm wall thickness is acceptable for this preliminary vacuum check.

## Tank Geometry

The tank geometry was determined based on the maximum liquid volume obtained from the ChemCAD simulation reports. From the tank’s liquid volume values, the maximum required liquid volume was found at the end of step 1, due to the addition of DMSO feed. This value is approximately Vmax=6 m3.

In order to provide an additional safety margin for bubbling and avoid operating the tank exactly at its maximum capacity, a larger geometrical volume was selected. The tank was designed as a vertical cylindrical vessel with a height of 2.5 m and a diameter of 2.0 m.

The volume of a cylindrical tank is calculated as:

V=πD24H  (15)

where:

- V = tank volume, in m³

- D = tank diameter, in m

- H = tank height, in m

Substituting with the selected dimensions:

V=π(2.0)24(2.5)=7.85 m3  (16)

Since this value is greater than the maximum required process volume of 6 m³, the tank geometry provides sufficient additional capacity for safe operation. The final selected tank dimensions are displayed in Table 6.3.

Final Dimensions of the selected tank.

| Parameter | Symbol | Value | Unit |
| --- | --- | --- | --- |
| Maximum required volume | Vmax | 6.00 | m3 |
| Selected tank diameter | D | 2.00 | m |
| Selected tank height | H | 2.50 | m |
| Selected tank volume | Vtank | 7.85 | m3 |

# Heat Exchangers

## Column Condenser Design

The condenser was designed as a counter-current, one-pass shell-and-tube heat exchanger used to condense the IPA-rich vapour leaving the top of the distillation column. A one-pass shell-and-tube heat exchanger is suitable for the column condenser because it avoids unnecessary mechanical complexity, reduces the number of flow reversals, and helps keep the pressure drop low. Since the required heat transfer area can be achieved with a simple tube bundle, this arrangement is sufficient (Coulson et al. 1999).

The total heat transfer area listed in the calculation corresponds to the total available outside tube surface area. The IPA properties used for the condensation-side calculation were obtained from ChemCAD’s reports, while the cooling-water properties were taken from literature (Cengel and Ghajar 2011). These are listed below in Table 7.1.

Material properties of isopropanol and cooling water.

| Property | Value | Unit |
| --- | --- | --- |
| thermal conductivity of IPA λIPA,film | 0.1235 | W/(m*K) |
| density of IPA ρIPA,film | 730.616 | kg/m3 |
| dynamic viscosity of IPA μIPA,film | 0.0005851 | N*s/m2 |
| latent heat of IPA rIPA,film | 680200 | J/kg |
| specific heat capacity of IPA, const. pressure cp,IPA | 3296.2 | J/(kg*K) |
| temperature of IPA Tdist | 80 | °C |
| thermal conductivity of water λw | 0.624 | W/(m*K) |
| density of water ρw | 994.1 | kg/m3 |
| dynamic viscosity of water μw | 0.000719 | N*s/m2 |
| specific heat capacity of water, const. pressure cp,w | 4175.5 | J/(kg*K) |
| temperature of inlet water Tw,in | 25 | °C |
| temperature of outlet water Tw,out | 40 | °C |

The main design equation used for the condenser was (Budapest University of Technology and Economics, Department of Building Service Engineering and Process Engineering 2024b):

Q=kAFΔTm  (17)

Rearranging for the required heat transfer area:

A=QkFΔTm  (18)

where Q is the condenser heat duty, k is the overall heat transfer coefficient, A is the total heat transfer area, F is the correction factor and ΔTm is the mean temperature difference. Since the temperature difference between the condensing vapour and cooling water changes along the exchanger, the logarithmic mean temperature difference was used:

ΔTlog=ΔT1−ΔT2lnΔT1ΔT2  (19)

The IPA vapour was assumed to condense at constant temperature since we are dealing with a phase change, while the cooling water was assumed to be heated up from 25°C (room temperature) to 50°C. Therefore:

ΔT1=TIPA−Tw,in=353.15−298.15=55 K

ΔT2=TIPA−Tw,out=353.15−323.15=30 K

Thus:

ΔTlog=55−30ln5530=41.24 K

The correction factor was calculated:

F=1

From the formulas indicated in the correction factor chart for a single shell pass heat exchanger (Figure 7.3).

The highest condenser heat duty obtained from the simulation was Q=957.7 MJ/h. Converting to watts:

Q=957.7×1063600=266027.78 W

Using the initially assumed overall heat transfer coefficient for phase change k=850 W/m2K (Kiss 2013), the first estimated heat transfer area was:

A=266027.78850×41.24=7.59 m2

This value was used only as an initial estimate. A corrected design was then obtained by calculating the individual heat transfer coefficients, the wall resistance, and the fouling resistance.

The clean overall heat transfer coefficient was calculated from the thermal resistance relationship (Perry and Green 2008):

1kclean=1αw,out+xtubeλwall+1αcond  (20)

where αw,out is the cooling-water heat transfer coefficient converted to the outside tube area, xtube is the tube wall thickness, λwall is the thermal conductivity of the carbon-steel wall, and αcond is the condensation-side heat transfer coefficient.

As it is a widely used, affordable material, it was decided to use carbon steel as the main material for the heat exchangers’ tubes and shell walls. The wall thermal conductivity (the thermal conductivity of carbon steel) was then taken to be λwall=45 W/mK (Perry and Green 2008).

The water-side Reynolds number was calculated using (Budapest University of Technology and Economics, Department of Building Service Engineering and Process Engineering 2024b):

Re=ρwvDinμ  (21)

The Prandtl number was calculated using (Budapest University of Technology and Economics, Department of Building Service Engineering and Process Engineering 2024b):

Pr=μcp,wλ  (22)

The Nusselt number used was (Budapest University of Technology and Economics, Department of Building Service Engineering and Process Engineering 2024b):

Nu=0.3Re0.5Pr1/3  (23)

The water-side heat transfer coefficient was then calculated from (Budapest University of Technology and Economics, Department of Building Service Engineering and Process Engineering 2024b):

αw,in=NuλwDin  (24)

Since the total heat transfer area was based on the outside tube surface, the water-side coefficient was converted to the outside area basis:

αw,out=αw,inDinDout  (25)

The calculated condenser heat transfer coefficients can be found in Table 7.2.

Condenser heat transfer coefficients.

| Parameter | Symbol | Value | Unit |
| --- | --- | --- | --- |
| Condensation-side coefficient | αcond | 1995.01 | W/m²K |
| Water-side coefficient, inside basis | αw,in | 1514.09 | W/m²K |
| Water-side coefficient, outside basis | αw,out | 1211.27 | W/m²K |
| Clean overall coefficient | kclean | 729.25 | W/m²K |
| Fouling resistance | Rf | 0.0005 | m²K/W |
| Dirty overall coefficient | kdirty | 534.40 | W/m²K |

Fouling resistance was obtained from literature (Perry and Green 2008).

The dirty overall heat transfer coefficient was calculated by adding the fouling resistance:

1Kdirty=1Kclean+Rf  (26)

kdirty=534.40 W/m2K

Using the dirty overall heat transfer coefficient, the final required heat transfer area was:

Acalc=QkdirtyΔTlog

Acalc=266027.78534.40×41.24=12.07 m2

Therefore, the required total condenser heat transfer area is 12.07 m2

The selected condenser geometry is shown in Table 7.3.

Selected and calculated condenser geometry.

| Parameter | Symbol | Value | Unit |
| --- | --- | --- | --- |
| Number of tubes | N | 48 | - |
| Tube outside diameter | Dout | 0.020 | m |
| Tube inside diameter | Din | 0.016 | m |
| Tube wall thickness | xtube | 0.002 | m |
| Tube pitch | pt | 0.025 | m |
| Tube length | L | 4.002 | m |
| Bundle diameter | Dbundle | 0.232 | m |
| Shell clearance | Cshell | 0.011 | m |
| Shell inside diameter | Dshell,in | 0.243 | m |
| Shell wall thickness | xshell | 0.0041 | m |
| Shell length | Lshell | 4.028 | m |
| Baffle spacing | - | 0.049 | m |

The tube outside diameter, tube thickness, tube pitch and shell clearance values were within those recommended by literature (Coulson et al. 1999). Similarly, the formulas for the bundle diameter, shell thickness, shell inside diameter and and shell length were given, respectively:

Dbundle=DoutNa1/b  (27)

For the selected one-pass square-pitch arrangement, the constants used were (Coulson et al. 1999):

- a=0.215

- b=2.207

xshell=PDshell,in2S+CA  (28)

where:

- P is the operating pressure (selected to be 600000 Pa)

- S is the allowable stress of the shell material, (120 MPa for carbon steel (ASME 2025a))

- CA is the corrosion allowance (0.0035 m, recommended by literature (Coulson et al. 1999))

Dshell,in=Dbundle+Cshell  (29)

Lshell=L+2Cshell+xshell  (30)

The number of tubes was arbitrarily selected to comply with length recommendations and the chosen array. Inversely, tube length was calculated with the following formula (Budapest University of Technology and Economics, Department of Building Service Engineering and Process Engineering 2024b):

L=ANπDout  (31)

The selected tube layout for both heat exchangers is a square, 90° tube array (Coulson et al. 1999) which is displayed in Figure 7.5. This arrangement is suitable for both the condenser and later, the reboiler because it gives a simple and compact tube bundle geometry, while also allowing easier mechanical cleaning between tube rows compared with triangular layouts. This is useful for a preliminary design, since the condenser may require periodic inspection or cleaning due to possible solvent impurities.

The available outside tube surface area was checked using:

A=NπDoutL=48π(0.020)(4.002)=12.07 m2

Therefore, the selected condenser geometry provides the required total heat transfer area.

The cooling-water mass flow rate was calculated from the sensible heat balance equation(Budapest University of Technology and Economics, Department of Building Service Engineering and Process Engineering 2024b):

Q=mwcp,wΔTw  (32)

Where ΔTw is the temperature difference of the cooling water (25 K). The calculated cooling-water mass flow rate was, after rearranging the previous equation:

mw=Qcp,wΔTw=2.55 kg/s

The main hydraulic results for the cooling water are given in Table 7.4.

Cooling water hydraulic results.

| Parameter | Symbol | Value | Unit |
| --- | --- | --- | --- |
| Water mass flow rate | mw | 2.55 | kg/s |
| Tube-side flow area | Atube | 0.00965 | m² |
| Water velocity | vw | 0.270 | m/s |
| Reynolds number | Rew | 5876.23 | - |
| Prandtl number | Prw | 4.81 | - |
| Nusselt number | Nuw | 38.82 | - |

As the IPA vapour changes phase, it will form a saturated liquid film along the surface of the tube walls. In order to accurately estimate the required temperature of the tube walls to carry out this heat transfer operation, the temperature difference across the film must be accounted for. The calculated wall and film temperatures can be observed in Table 7.5.

Condenser wall and film temperatures.

| Parameter | Symbol | Value [°C] |
| --- | --- | --- |
| Real mean wall temperature | Twall,real | 68.62 |
| Calculated mean wall temperature | Twall,calc | 70.00 |
| Mean film temperature | Tfilm | 75.00 |
| Film temperature difference | ΔTfilm | 10.00 |

The value of the film temperature difference was chosen by iteration, performed until the calculated wall temperature was close to the real wall temperature. A normal temperature difference interval between the film and wall temperatures is of around 5 to 15°C.

The calculated mean wall temperature could be obtained from the mean temperature of the condensed vapour or distillate Tdist, and the film temperature difference:

Twall,calc=Tdist−ΔTfilm  (33)

The real, mean wall temperature was calculated to be (Budapest University of Technology and Economics, Department of Building Service Engineering and Process Engineering 2024b):

Twall,real=Tdist−kdirtyαcond(Tdist−Tw)  (34)

Where Tw is the mean temperature of the water.

The small difference between the real and calculated wall temperatures shows that the assumed film temperature difference is acceptable for the preliminary condenser design. All other calculated parameters are realistic for the scale of the process and adequate for manufacturing.

## Column Reboiler Design

The reboiler was designed as a kettle reboiler. Although it is not a traditional shell-and-tube heat exchanger in the same way as the condenser, it still uses a tube bundle to transfer heat from condensing steam to the process liquid. The reboiler was designed as a two-pass heat exchanger arrangement, with the listed heat transfer area representing the total heat transfer area.

A kettle reboiler is more suitable for this project than a jacketed tank or internal coils since they are mainly used for heating agitated vessels, where the liquid temperature changes with time. In this case, the required duty is continuous vaporization at the bottom of the column. A kettle reboiler provides a submerged tube bundle with a large heat transfer surface, allowing the liquid to boil directly on the shell side while also providing enough volume for vapour-liquid disengagement (Perry and Green 2008).

The DMSO-rich liquid properties were obtained from ChemCAD using the relevant column-bottom composition, while the steam-side properties were obtained from a saturated steam table (Cengel and Ghajar 2011). The obtained values are showcased below, in Table 7.6.

Material properties of DMSO and steam.

| Property | Value | Unit |
| --- | --- | --- |
| vapor density of DMSO ρDMSO,vap | 2.1 | kg/m3 |
| liquid density of DMSO ρDMSO | 927.4 | kg/m3 |
| latent heat of DMSO rDMSO | 605000 | J/kg |
| specific heat capacity of DMSO cp,DMSO | 2323 | J/(kg*K) |
| surface tension of DMSO σDMSO | 0.022 | N/m |
| thermal conductivity of DMSO λDMSO | 0.1 | W/(m*K) |
| dynamic viscosity of DMSO μDMSO | 0.00042 | N*s/m2 |
| temperature of DMSO TDMSO | 190 | °C |
| thermal conductivity of steam λst,film | 0.65455 | W/(m*K) |
| liquid density of steam ρst,film | 848.7 | kg/m3 |
| vapor density of steam ρst,v | 10.286 | kg/m3 |
| dynamic viscosity of steam μst,film | 0.0001262 | N*s/m2 |
| latent heat of steam rst | 1887700 | J/kg |
| specific heat capacity of steam cp,st | 4571.5 | J/(kg*K) |
| temperature of steam Tst | 220 | °C |

In this case, it was not necessary to calculate the logarithmic temperature difference, since the steam temperature was chosen and the temperature of the bottoms is known (see Figure 7.6). Furthermore, the heat duty is known.

The steam table used can be seen in Figure 7.7. A saturated steam pressure of 23.2 bar corresponds to the selected saturated steam temperature. A design steam pressure of 24 bar was selected for wall thickness selection, to provide an extra margin of safety.

The design equation used for the reboiler was:

Q=kAFΔT  (35)

Rearranging for heat transfer area:

A=QkFΔT  (36)

The DMSO-rich liquid temperature was taken to be TDMSO=190∘C. The heating steam temperature was then chosen as Tst=220∘C, for an adequate temperature difference of ΔT=30 K. This will allow for proper heat transfer without overheating the DMSO, which properties are to be taken as reference since it possesses the highest boiling resistance. For stages where DMSO does not make up most of the bottom liquid, the pressure of steam (and therefore the temperature) is to be decreased with a control system. At higher temperatures, a peak and then a rapid decrease in the value of the heat transfer coefficient can be observed (Fonyó and Fábry 2004). An illustration of this phenomenon with water as an example is illustrated in Figure 7.7.

The reboiler heat duty was Q=1000 MJ/h. Converting to watts:

Q=1000×1063600=277777.78 W

Using the initially assumed overall heat transfer coefficient for phase change k=850 W/m2K, the preliminary heat transfer area was:

A=277777.78850×30=10.89 m2

This initial estimate was corrected by calculating the individual boiling-side and steam-condensation-side heat transfer coefficients.

The boiling-side heat transfer coefficient for the process was calculated using the pool-boiling correlation (Perry and Green 2008):

αboiling=0.225Q⋅cpA⋅r0.69P⋅λσ0.31ρlρv0.33  (37)

where:

- αboiling = boiling-side heat transfer coefficient, W/m²K

- Q = heat duty, W

- A = heat transfer area, m²

- cp = liquid specific heat capacity, J/kgK

- r = latent heat of vaporization, J/kg

- P = operating pressure, Pa

- λ = liquid thermal conductivity, W/mK

- σ = surface tension, N/m

- ρl = liquid density, kg/m³

- ρv = vapour density, kg/m³

These values for DMSO were obtained from ChemCAD reports and listed at the beginning of the subchapter.

αshell=2261.06 W/m2K

The steam-side condensation coefficient was calculated using the filmwise-condensation method. The inside coefficient was first calculated and then converted to the outside tube surface basis (Budapest University of Technology and Economics, Department of Building Service Engineering and Process Engineering 2024b):

αst,out=αst,inDinDout

The calculated reboiler heat transfer coefficients are displayed in Table 7.7.

Reboiler heat transfer coefficients.

| Parameter | Symbol | Value | Unit |
| --- | --- | --- | --- |
| Boiling-side coefficient | αshell | 2261.06 | W/m²K |
| Steam condensation coefficient, inside basis | αst,in | 12152.33 | W/m²K |
| Steam condensation coefficient, outside basis | αst,out | 10194.74 | W/m²K |
| Clean overall coefficient | kclean | 1708.92 | W/m²K |
| Fouling resistance | Rf | 0.00026 | m²K/W |
| Dirty overall coefficient | kdirty | 1183.20 | W/m²K |

The clean overall heat transfer coefficient was calculated as:

1kclean=1αst,out+xtubeλwall+1αshell

The dirty overall heat transfer coefficient was calculated by including the fouling resistance (Perry and Green 2008):

1kdirty=1Kclean+Rf

kdirty=1183.20 W/m2K

The correction factor for the two-pass heat exchanger was possible to compute:

F=1

From the correction factor diagram for a two-pass heat exchanger (see Figure 7.9).

Using the dirty overall coefficient, the corrected reboiler heat transfer area was:

Acalc=QkdirtyFΔT=277777.781183.20×30=7.83 m2

Therefore, the required total reboiler heat transfer area is 7.83  m2

The selected reboiler geometry is shown below, in Table 7.8. The same formulas and values obtained from literature used in subchapter 7.1, were applied in this subchapter (Coulson et al. 1999). Tube wall thickness was selected to be compatible with high pressure steam.

Selected and calculated reboiler geometry.

| Parameter | Symbol | Value | Unit |
| --- | --- | --- | --- |
| Number of tubes | N | 28 | - |
| Tube outside diameter | Dout | 0.0250 | m |
| Tube inside diameter | Din | 0.0210 | m |
| Tube wall thickness | xtube | 0.00202 | m |
| Tube pitch | pt | 0.0313 | m |
| Tube length | L | 1.78 | m |
| Bundle diameter | Dbundle | 0.227 | m |
| Shell clearance | Cshell | 0.011 | m |
| Shell inside diameter | Dshell,in | 0.477 | m |
| Shell wall thickness | xshell | 0.00493 | m |

The tube length was calculated from the total required heat transfer area, dividing the result by 2 since the configuration will have two tube passes. This means that the length of the tubes need to be half the size as for a single pass, while the shell diameter will double (Budapest University of Technology and Economics, Department of Building Service Engineering and Process Engineering 2024b). The formula used to compute the tube length was:

L=A2NπDout=7.832(28)π(0.0250)=1.78 m  (38)

The steam mass flow rate was calculated from the latent heat balance formula (Budapest University of Technology and Economics, Department of Building Service Engineering and Process Engineering 2024b):

mst=Qrst=277777.781887700=0.147 kg/s  (39)

The main steam-side values related to flow rate were also computed, using the same formula for flow cross-sectional area and velocity as in the previous chapter. Results are listed in Table 7.9

Steam flow rate parameters.

| Parameter | Symbol | Value | Unit |
| --- | --- | --- | --- |
| Steam mass flow rate | mst | 0.147 | kg/s |
| Tube-side flow area | Atube | 0.00485 | m² |
| Steam velocity | vst | 2.95 | m/s |

Similarly, film and wall mean temperature results could be obtained by utilizing the same formulas as in the case of the condenser, but assuming that this time it will be steam forming a film around the tubes as it condenses. The calculated and selected film and wall temperatures are found in Table 7.10.

Reboiler wall and film temperatures.

| Parameter | Symbol | Value [°C] |
| --- | --- | --- |
| Real wall temperature | Twall,real | 204.30 |
| Calculated wall temperature | Twall,calc | 205.00 |
| Film temperature | Tfilm | 212.50 |
| Film temperature difference | ΔTfilm | 15.00 |

The small difference between the real and calculated wall temperatures confirms that the assumed film temperature is acceptable for the preliminary kettle reboiler calculation.

All calculated parameters are realistic for the scale of the process and adequate for the manufacturing of parts for this reboiler.

## Selection of Heat Exchanger Construction Types

The condenser was selected as a conventional one-pass shell-and-tube heat exchanger. Since the exchanger is relatively small and the temperature difference is moderate, a fixed-tube-sheet construction is suitable. For the front-end stationary head, a removable channel and cover is selected so that the tube side can be accessed for inspection and cleaning. The selected front-end type is therefore TEMA type A (Coulson et al. 1999). The shell is selected as a one-pass shell, corresponding to TEMA shell type E. The rear-end head is selected as a fixed-tube-sheet rear channel with removable cover, corresponding to TEMA rear-end type L. Therefore, the selected condenser construction is:

AEL

where:

- A = front-end stationary head with removable channel and cover

- E = one-pass shell

- L = fixed-tube-sheet rear end with removable cover

This construction is appropriate for a normal one-pass condenser because it is mechanically simple, allows tube-side access, and provides the required heat transfer area.

The reboiler was selected as a kettle-type reboiler rather than a conventional shell-and-tube heat exchanger. For this reason, the shell type must correspond to a kettle arrangement. The selected shell type is therefore TEMA type K (Coulson et al. 1999). Since the reboiler uses a two-pass tube arrangement and must allow thermal expansion of the tube bundle, a U-tube rear-end construction is suitable. The selected rear-end type is TEMA type U. For the front-end stationary head, a bonnet-type head is selected because the tube-side fluid is condensing steam, which is relatively clean compared with process liquids. The selected front-end type is therefore TEMA type B. Therefore, the selected reboiler construction is:

BKU

where:

- B = bonnet-type front-end stationary head

- K = kettle-type shell

- U = U-tube rear-end construction

This construction is appropriate for the kettle reboiler because the kettle shell provides space for boiling and vapour-liquid disengagement, while the U-tube arrangement allows thermal expansion and provides the required two-pass tube-side flow path.

# Heat Recycling

Distillation is an energy-intensive separation process because heat must be supplied to the reboiler while it is simultaneously removed from the overhead vapour in the condenser. In the original configuration, a significant part of the thermal energy contained in the top vapour is rejected during condensation. For this reason, heat recycling was studied as a way to reduce the external heating demand of the column and improve the energetic efficiency of the process.

In this chapter, two heat recovery methods are investigated. The first method is the pre-heater method, in which the vapour leaving the top of the column during the IPA distillation is used as a heat source in a heat exchanger. This recovered heat is transferred to the DMSO feed stream before it enters the column, eliminating external energy required to pre-heat the feed. This method was simulated in ChemCAD using a simple heat exchanger model, as can be seen in Figure 8.1.

The second method is the heat pump method, in which the top vapour from the column is compressed in order to increase its temperature and make it suitable for heat recovery at the bottom of the column. The simulated arrangement includes two compressors with an inter-cooler, a heat exchanger, an expansion valve, and a second heat exchanger. In this configuration, heat is extracted from the compressed overhead vapour and used to help heat the column bottom, reducing the required reboiler heat load. This method was studied using a dynamic ChemCAD simulation. The top-vapour composition was introduced into the model at time steps of 0.05 h over the distillation operation.

The purpose of comparing these two methods is to determine how effectively the available overhead-vapour heat can be reused in the process. The pre-heater method represents a simpler and more direct heat-recovery option, while the heat pump method represents a more complex but potentially more powerful heat-integration strategy. Both methods are evaluated based on their ability to reduce external heat requirements and improve the energetic performance of the distillation process.

## Pre-heater Method

In the pre-heater method, the heat contained in the vapour leaving the top of the distillation column is used to pre-heat the DMSO feed before it enters the column. This reduces the amount of external heat required to bring the solvent feed to the selected inlet temperature. The simulation was carried out in ChemCAD using a heat exchanger, where the hot side corresponds to the overhead vapour and the cold side corresponds to the feed stream.

We used the main design equation for the heat exchangers is, and the logarithmic mean temperature difference was calculated from the two terminal temperature differences:

ΔTlog=ΔT1−ΔT2lnΔT1ΔT2

where:

ΔT1=T1,1−T2,1

ΔT2=T1,2−T2,2

We need to heat up the feed to increase its temperature from 20°C (room temperature) to 62°C. Since the vapour, which is almost pure IPA, experiences a phase change (condensation), its temperature can be assumed to remain constant (Budapest University of Technology and Economics, Department of Building Service Engineering and Process Engineering 2024a). The temperatures used in the pre-heater calculation, in Kelvin, are shown in Table 8.1.

Pre-heater temperatures.

| Parameter | Symbol | Value | Unit |
| --- | --- | --- | --- |
| Hot inlet temperature | T1,1 | 355.55 | K |
| Hot outlet temperature | T1,2 | 355.55 | K |
| Cold inlet temperature | T2,1 | 293.15 | K |
| Cold outlet temperature | T2,2 | 335.15 | K |

Replacing the logarithmic mean temperature difference formula with these values:

ΔTlog=37.57 K

The value of the total heat duty of this process, obtained from the ChemCAD heat-exchanger simulation, was:

Q=45.1324 MJ/h

This value was converted into watts:

Q=45.1324×1063600=12536.78 W

The calculated temperature differences and heat duty are summarized below:

Pre-heater heat duty.

| Parameter | Symbol | Value | Unit |
| --- | --- | --- | --- |
| Terminsl T difference 1 | ΔT1 | 62.40 | K |
| Terminal T difference 2 | ΔT2 | 20.40 | K |
| LMTD | ΔTlog | 37.57 | K |
| Heat duty | Q | 45.13 | MJ/h |

Three possible heat transfer areas were calculated using different overall heat transfer coefficients. The first value, Kvap=850 W/m2K, represents the assumed overall heat transfer coefficient for a change of phase in a heat exchanger. The second value, Kdist=570 W/m2K (Kiss 2013), represents an estimation of the required heat transfer coefficient for the condensation of IPA vapour. The third value, Kcalc=534.4 W/m2K, represents the calculated overall heat transfer coefficient used for the final conservative sizing. This is the heat transfer coefficient that was calculated for the main condenser of this column.

The highest calculated heat transfer area is obtained using the lowest overall heat transfer coefficient:

For Kcalc=534.4 W/m2K:

Acalc=12536.78534.4×37.57

Acalc=0.6245 m2

Therefore, the selected required heat transfer area for the pre-heater is 0.6245 m2.

The pre-heater therefore allows the feed to enter the distillation column at the selected feed temperature of 62°C using recovered heat from the overhead vapour. This decreases the amount of additional external energy that would otherwise be required to heat the feed before it enters the column. Not only would it efficiently save on energy, but also on costs. It provides a simple and practical way to recycle part of the heat contained in the overhead vapour, while its required size area is relatively small.

## Heat Pump Method

The second heat recycling method studied was the heat pump method. In this configuration, the vapour leaving the top of the distillation column is compressed. This makes the overhead vapour suitable for heat recovery at a higher temperature level. The recovered heat is then transferred to the bottom of the column, helping the reboiler supply the energy required for vaporization.

Before the final arrangement was selected, different compression ratios were investigated using a single compressor. The purpose of these trials was to determine whether the overhead vapour could be compressed sufficiently in one step to reach a useful temperature level for reboiler heat recovery. Several pressure ratios were tested, but the results showed that a single compressor was not suitable. Lower compression ratios did not increase the temperature enough for useful heat recovery over a sufficient part of the operation, while higher compression ratios produced excessively high outlet temperatures and unsuitable vapour-fraction behavior. Therefore, the final selected configuration used two compressors with an inter-cooler between them. Each compressor was operated with a compression ratio of 1:4.

The simulated heat pump arrangement in ChemCAD included two compressors, an inter-cooler between them, a first heat exchanger with a set heat duty of -1000 MJ/h to represent the heat taken out by the reboiler (the heat duty of the reboiler for the all distillation simulations was set to 1000 MJ/h; therefore, theoretically, this amount of energy per hour that will be demanded), an expansion valve, and a second heat exchanger that was used to document the amount of remaining energy that could be recovered and used for other purposes. The inter-cooler was required to reduce the vapour temperature between compression stages and to avoid excessively high temperatures after the second compressor. The two-stage compression arrangement also made the heat pump behavior more stable than the single-compressor cases.

The simulation was carried out dynamically using ChemCAD data maps and Excel coding. The input vapour composition and temperature at each time step was obtained from ChemCAD reports for the top vapour stream, these values were out-putted as a large excel table. Our excel file was programmed to transfer the data values to a specific cell in the document every 0.05 h (see code in Appendix). ChemCAD then read these values through a data map and used them as inputs for the heat pump simulation. In the same way, the output values from ChemCAD were exported back into Excel every 0.05 h and stored in a second table.

The main outputs recorded were the temperature after compression, vapour fraction after compression, temperature after the first heat exchanger, vapour fraction after the first heat exchanger, heat duty of the second heat exchanger, inter-cooler heat duty, compressor duties, cooling-water outlet temperature, and logarithmic mean temperature difference. Although the full simulation was carried out over the complete operation, the heat pump was found to be suitable only during the first 4.5 h of the distillation. Therefore, only this time interval was considered for the useful heat recovery evaluation.

### Input Vapour Stream

The input vapour stream composition changes continuously during the batch distillation. The dynamic heat pump simulation therefore required the vapour composition to be updated at each time step. The table below shows representative values from the input table, using 0.5 h intervals up to 4.5 h. After 4.5 hours had passed, the output results from the simulation were not good enough for efficient heat recycling anymore.

Heat pump input values.

| Time [h] | R | IPA-L [kmol/h] | H₂O-L [kmol/h] | DMSO-L [kmol/h] | T [°C] | IPA-V [kmol/h] | H₂O-V [kmol/h] | DMSO-V [kmol/h] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.50 | 4 | 4.7279 | 0.0026 | 0.000053 | 81.82 | 23.6395 | 0.0129 | 0.000263 |
| 1.00 | 4 | 4.7091 | 0.0023 | 0.000053 | 83.57 | 23.5455 | 0.0116 | 0.000263 |
| 1.50 | 4 | 4.6917 | 0.0021 | 0.000053 | 85.44 | 23.4585 | 0.0106 | 0.000263 |
| 2.00 | 4 | 4.6736 | 0.0020 | 0.000053 | 87.43 | 23.3680 | 0.0098 | 0.000263 |
| 2.50 | 4 | 4.6538 | 0.0018 | 0.000053 | 89.56 | 23.2690 | 0.0092 | 0.000263 |
| 3.00 | 4 | 4.6307 | 0.0017 | 0.000053 | 91.82 | 23.1535 | 0.0085 | 0.000263 |
| 3.50 | 4 | 4.6043 | 0.0016 | 0.000052 | 94.26 | 23.0215 | 0.0080 | 0.000262 |
| 4.00 | 4 | 4.5732 | 0.0015 | 0.000052 | 96.89 | 22.8661 | 0.0075 | 0.000262 |
| 4.50 | 4 | 4.5364 | 0.0014 | 0.000052 | 99.75 | 22.6821 | 0.0070 | 0.000262 |

As shown in Table 5, the IPA vapour flow rate slowly decreases during the first 4.5 h, while the vapour temperature increases from approximately 80°C to nearly 100°C. The DMSO vapour flow rate remains very small compared with the IPA vapour flow rate. This is expected because DMSO has a much higher boiling point and remains mainly in the liquid phase.

### Heat Pump Output Results

The table below shows the main output values obtained from the ChemCAD heat pump simulation. The table includes only the useful operating region up to 4.5 h.

Heat pump output values.

| Time [h] | T comp. [°C] | VF comp. | T HX1 [°C] | VF HX1 | HX2 duty [MJ/h] | IC duty [MJ/h] | C1 [MJ/h] | C2 [MJ/h] | LMTD [K] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.50 | 177.21 | 1.00 | 114.36 | 0.00 | 164.31 | 23.97 | 97.18 | 99.29 | 89.36 |
| 1.00 | 177.21 | 1.00 | 113.62 | 0.00 | 159.62 | 23.87 | 96.79 | 98.89 | 89.41 |
| 1.50 | 177.21 | 1.00 | 112.93 | 0.00 | 155.30 | 23.77 | 96.43 | 98.52 | 89.46 |
| 2.00 | 177.20 | 1.00 | 112.20 | 0.00 | 150.79 | 23.68 | 96.05 | 98.13 | 89.51 |
| 2.50 | 177.20 | 1.00 | 111.39 | 0.00 | 145.88 | 23.57 | 95.64 | 97.71 | 89.57 |
| 3.00 | 177.20 | 1.00 | 110.45 | 0.00 | 140.18 | 23.45 | 95.17 | 97.23 | 89.64 |
| 3.50 | 177.20 | 1.00 | 109.36 | 0.00 | 133.65 | 23.32 | 94.62 | 96.67 | 89.71 |
| 4.00 | 177.20 | 1.00 | 108.05 | 0.00 | 125.97 | 23.16 | 93.98 | 96.01 | 89.80 |
| 4.50 | 177.20 | 1.00 | 106.47 | 0.00 | 116.89 | 22.97 | 93.22 | 95.24 | 89.90 |

The results show that the vapour fraction after compression remains equal to 1.00 during the useful period, meaning that the stream remains fully vapour after compression. After the first heat exchanger, the vapour fraction becomes 0.00, meaning that the compressed vapour is condensed and heat is successfully extracted. This confirms that the heat pump arrangement works properly during the first 4.5 h.

The time-zero value is treated as a start-up point and is not considered representative of stable heat pump operation, therefore 0.05 h is the initial time value. The heat duty of the second heat exchanger decreases in magnitude during the useful operating period, from approximately 169.40 MJ/h at 0.05 h to 116.89 MJ/h at 4.5 h. This shows that the amount of useful recoverable heat decreases as the batch distillation progresses, and that the method would be most efficient when only applied during the first 4.5 hours. During the time interval 7.5 h - 9.5 h, the heat duty becomes negative, meaning that the heat would be lost and making recovery definitely not viable during this range. The compressor duties also decrease slightly over time, while the logarithmic mean temperature difference remains almost constant, close to 89–90 K. Furthermore, the temperature difference between the input vapour and output liquid after heat recovery decreases to less than 10 degrees Celsius, which is not suitable for efficient heat transfer applications (Kiss 2013), (Kemp 2007).

### Useful Heat Pump Operation

The useful heat pump operating period can be summarized:

Useful heat pump period.

| Parameter | Value | Unit |
| --- | --- | --- |
| Start time | 0.05 | h |
| End time | 4.50 | h |
| Max. HX2 duty | 169.40 | MJ/h |
| Max. inter-cooler duty | 24.08 | MJ/h |
| Max. compressor 1 duty | 97.61 | MJ/h |
| Max. compressor 2 duty | 99.72 | MJ/h |
| Avg. LMTD | 89.57 | K |

Summary of the heat pump’s energy demand and output.

| Equipment | Max. duty [MJ/h] |
| --- | --- |
| Compressor 1 | 97.61 |
| Compressor 2 | 99.72 |
| Inter-cooler | 24.08 |
| Heat exch. 2 | 169.4 |

Using the maximum compressor duties as a conservative estimate, the energy needed to operate the heat pump for 4.5 h is:

(97.61+99.72)×4.5=887.985 MJ  (40)

However, the heat pump will replace the duty of the reboiler duty during this time period, which was assumed to be 1000 MJ/h throughout the entire process. Therefore, the total energy is:

1000×4.5=4500 MJ  (41)

The net energy saving is therefore approximately:

4500−887.985=3612.015 MJ  (42)

This means that, from an energy-balance point of view, the heat pump is viable since the energy recovered for the reboiler is much greater than the compressor energy required. This method can be considered energetically beneficial as a partial heat-recovery option, but its final practical viability must also consider its higher equipment complexity and capital cost, since it requires two compressors, an inter-cooler, an expansion valve, and additional heat exchangers.

For future cost estimation, the inter-cooler’s maximum heat duty and heat transfer area were used.

Conversion to watts:

Qmax=24.08×1063600=6688.01 W

Assuming cooling water is heated from 25°C to 50°C:

mw=QcpΔT

Using cp=4180 J/kgK and ΔT=25 K:

mw=6688.014180×25=0.0640 kg/s

mw=230.40 kg/h

The logarithmic mean temperature difference calculated was:

ΔTlog=88.46 K

Assuming the inter-cooler is a single pass heat exchanger, the correction factor F was calculated to be 1 from correction factor graph for a single-pass heat exchanger (Coulson et al. 1999).

Using K=150 W/m2K (Perry and Green 2008), The heat transfer area was calculated using the heat transfer area equation for heat exchangers:

A=6688.01150×88.46=0.501 m2

## Method Comparison

For the pre-heater method, the recovered overhead-vapour heat is used to heat the feed before it enters the column. The total recovered energy is 45×9.05=407.25 MJ. Therefore, only about (50−45)×9.05=45.25 MJ of additional external feed-heating energy would still be required. In comparison, the heat pump method can replace 1000×4.5=4500 MJ of reboiler energy during its useful 4.5 h operating period. It requires more energy to operate, but the net energy saving is around 3600 MJ. Therefore, the heat pump method is clearly superior from a net energy-saving perspective.

However, the heat pump method is much more complex and requires additional equipment, including two compressors, an inter-cooler, an expansion valve, and additional heat exchangers. Its main disadvantage is that it is only useful for a limited part of the batch operation and requires significant compressor work. For this reason, the heat pump method may be useful as a partial heat-recovery option, but it is more complicated and less broadly applicable than the pre-heater method, as well as more costly.

# Material Recovery

After the main batch extractive distillation step, two secondary run-off streams are produced: an IPA/water run-off and a water/DMSO run-off. These streams do not meet the purity requirements to be considered final products, but they still contain recoverable amounts of IPA, water, and DMSO. Therefore, instead of treating them as waste, their re-integration into later batches was studied in order to reduce material losses and improve the overall efficiency of the process.

Two recycling strategies were compared using ChemCAD simulations. In both cases, the results of one batch were used as input for the next batch. Batch 1 corresponds to the original reference batch, while Batches 2–6 represent five consecutive recycle batches. For each batch, the total moles, mass, purity of the run-off streams, IPA distillate purity, distillate mass, elapsed time, and DMSO bottom-product purity were recorded.

In the first method, only the IPA/water run-off is added to the pot charge of the next batch, while the recovered DMSO bottom product from the previous distillation is used as the solvent feed for the next batch. Since the amount of recovered DMSO is lower than the amount required for a complete new batch, the missing amount would be replaced in practice by externally supplied, theoretically pure DMSO. However, in the simulation, the additional pure DMSO was not included in the purity calculation. Instead, the purity of the recovered bottom DMSO was used as the next solvent-feed purity, representing the worst-case scenario. The water/DMSO run-off is not directly recycled in this method and is instead kept for later recovery distillation.

In the second method, both the IPA/water run-off and the water/DMSO run-off are added directly to the pot charge of the next batch. This method has the advantage of immediately reusing all run-off material, but it also introduces more water and DMSO into the next batch, which can influence the charge composition, separation time, and simulation convergence.

Note: in order to fit the tables into the page frame, some values were rounded up to one decimal point. However, the plots and final conclusions include more precise values.

## Method 1, Water/IPA Run-off Re-integration

In the first recycling method, the IPA/water run-off from each batch was added to the pot charge of the following batch. The DMSO bottom product from the previous batch was reused as the next batch’s solvent feed. The water/DMSO run-off was not added directly to the following batch, but was instead collected for later recovery. This approach aims to recover IPA-containing material without continuously increasing the amount of DMSO in the pot charge.

The initial charge values used for this recycle sequence were 41.18112 kmol IPA and 21.21452 kmol water. During the ChemCAD simulations, some operating adjustments were required to maintain convergence and product purity. For Batch 2, the reflux ratio in step 3 was changed from 4 to 5. For Batch 5, the reflux ratio in step 3 was further changed from 5 to 6. These changes were necessary to keep the distillation within the required purity range as the recycled material influenced the batch composition.

### IPA/Water Run-off and IPA Distillate Results

Results of the first material recovery method.

| B | IPA RO | H₂O RO | xIPA RO | mRO | nIPA Chg. | nH₂O Chg. | xH₂O Chg. | xIPA Chg. | xIPA Dist. | mDist. | t |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | kmol | kmol | mol frac. | kg | kmol | kmol | mol frac. | mol frac. | mol frac. | kg | h |
| 1 | 1.30 | 5.29 | 0.1975 | 173.6 | 42.48 | 26.51 | 0.3842 | 0.6158 | 0.9985 | 2397.4 | 16.1 |
| 2 | 1.71 | 7.04 | 0.1957 | 229.9 | 42.90 | 28.26 | 0.3971 | 0.6029 | 0.9989 | 2477.9 | 18.2 |
| 3 | 1.68 | 6.71 | 0.1999 | 221.6 | 42.86 | 27.92 | 0.3945 | 0.6055 | 0.9985 | 2477.2 | 18.5 |
| 4 | 1.65 | 6.73 | 0.1971 | 220.6 | 42.83 | 27.94 | 0.3948 | 0.6052 | 0.9985 | 2477.1 | 18.8 |
| 5 | 1.80 | 7.36 | 0.1963 | 240.6 | 42.98 | 28.57 | 0.3993 | 0.6007 | 0.9990 | 2466.7 | 19.5 |
| 6 | 1.74 | 7.04 | 0.1977 | 231.1 | 42.92 | 28.25 | 0.3970 | 0.6030 | 0.9987 | 2479.4 | 19.8 |

The IPA distillate purity remained consistently above the required value in all six batches. The average IPA distillate purity was 0.9987 mol fraction, while the total recovered IPA distillate mass was 14775.70 kg. The total operating time for the six batches was 111.10 h. A slight increase in operating time can be observed from Batch 1 to Batch 6, which is expected because the recycled IPA/water run-off changes the initial pot composition of each new batch.

### Water/DMSO Run-off Obtained During Method 1

Water/DMSO run-off obtained during method 1.

| B | H₂O | DMSO | xH₂O | xDMSO | mTotal |
| --- | --- | --- | --- | --- | --- |
|  | kmol | kmol | mol frac. | mol frac. | kg |
| 1 | 3.29 | 15.92 | 0.1713 | 0.8287 | 1302.9 |
| 2 | 2.47 | 15.75 | 0.1356 | 0.8644 | 1275.1 |
| 3 | 2.57 | 16.01 | 0.1382 | 0.8618 | 1297.5 |
| 4 | 2.60 | 15.98 | 0.1398 | 0.8602 | 1295.3 |
| 5 | 2.06 | 15.48 | 0.1176 | 0.8824 | 1246.8 |
| 6 | 2.00 | 15.56 | 0.1139 | 0.8861 | 1252.1 |

The water/DMSO run-off contains a significant amount of DMSO, with an average DMSO mole fraction of 0.8639. Since this stream is not directly returned to the next batch in Method 1, it remains available for a later recovery distillation. This prevents additional DMSO accumulation in the main IPA recovery process, but requires an additional recovery step.

### DMSO Bottom Product Obtained During Method 1

DMSO bottom product obtained during method 1.

| B | DMSO | H₂O | xDMSO | xH₂O | mTotal |
| --- | --- | --- | --- | --- | --- |
|  | kmol | kmol | mol frac. | mol frac. | kg |
| 1 | 51.96 | 0.0255 | 0.9995 | 0.00049 | 4060.02 |
| 2 | 53.20 | 0.0253 | 0.9995 | 0.00047 | 4157.48 |
| 3 | 53.70 | 0.0252 | 0.9995 | 0.00047 | 4195.97 |
| 4 | 53.73 | 0.0256 | 0.9995 | 0.00048 | 4198.89 |
| 5 | 53.85 | 0.0253 | 0.9995 | 0.00047 | 4207.86 |
| 6 | 54.14 | 0.0248 | 0.9995 | 0.00046 | 4230.33 |

The DMSO bottom product remained highly pure throughout the recycle sequence, with an average DMSO mole fraction of 0.9995. This confirms that the recovered bottom DMSO can be reused as solvent feed in the following batch. However, the recovered amount is not enough to supply a full new batch, so external pure DMSO would still be required in practice.

The water/IPA run-off re-integration method is stable and effective. It maintains the IPA distillate purity above the required specification, gives a high total recovered distillate mass, and produces a very pure DMSO bottom stream suitable for reuse. Its main disadvantage is that the water/DMSO run-off is not immediately recycled and must be treated separately later. However, this also makes the main distillation sequence easier to control, since DMSO does not accumulate as strongly in the next batch charge.

## Method 2, Total Run-off Re-integration

In the second recycling method, both the IPA/water run-off and the water/DMSO run-off were added directly to the next pot charge. This means that all run-off material from the previous batch was immediately reused in the following batch. The purpose of this method was to determine whether direct recycling of all secondary material streams could improve material recovery and reduce the need for a separate recovery step.

The same initial charge values were used as in Method 1: 41.18112 kmol IPA and 21.21452 kmol water. Since both run-off streams were recycled, the DMSO content in the pot charge increased compared with Method 1. This made the simulation more difficult to converge in some cases. In Batch 2, the reflux ratio in step 3 was changed from 4 to 5. In Batch 3, the stop value in step 3 was changed from 0.9995 to 0.9997. In Batch 4, the reflux ratio in step 3 was increased from 5 to 6. In Batch 5, the reflux ratio was changed back from 6 to 5. These adjustments show that Method 2 required more operational changes than Method 1.

### Combined IPA/Water/DMSO Run-off and IPA Distillate Results

Results of the second material recovery method.

| B | IPA RO | H₂O RO | DMSO RO | mRO | nIPA Chg. | nH₂O Chg. | xIPA Chg. | xH₂O Chg. | xDMSO Chg. | xIPA Dist. | mDist. | t |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | kmol | kmol | kmol | kg | kmol | kmol | mol frac. | mol frac. | mol frac. | mol frac. | kg | h |
| 1 | 1.30 | 8.58 | 15.92 | 1476.5 | 42.48 | 29.80 | 0.4817 | 0.3378 | 0.1805 | 0.9985 | 2397.4 | 16.1 |
| 2 | 1.98 | 11.11 | 19.26 | 1824.0 | 43.17 | 32.32 | 0.4556 | 0.3411 | 0.2033 | 0.9989 | 2468.5 | 19.7 |
| 3 | 2.10 | 11.9 | 20.43 | 1936.6 | 43.28 | 33.08 | 0.4471 | 0.3417 | 0.2111 | 0.9988 | 2468.5 | 20.7 |
| 4 | 2.06 | 11.23 | 20.26 | 1909.2 | 43.24 | 32.44 | 0.4507 | 0.3381 | 0.2112 | 0.9985 | 2478.1 | 21.9 |
| 5 | 2.03 | 11.77 | 20.93 | 1969.6 | 43.21 | 32.98 | 0.4449 | 0.3396 | 0.2155 | 0.9985 | 2477.4 | 20.8 |
| 6 | 2.19 | 12.33 | 21.00 | 1994.2 | 43.37 | 33.54 | 0.4429 | 0.3426 | 0.2145 | 0.9989 | 2465.9 | 21.0 |

The IPA distillate purity remained above the target in all batches, with an average value of 0.9987 mol fraction. However, the total distillate mass was slightly lower than in Method 1, with 14755.93 kg recovered over the six batches. The total operating time increased to 120.40 h, which is 9.30 h longer than Method 1.

### DMSO Bottom Product Obtained During Method 2

DMSO bottom product obtained during method 2.

| B | DMSO | H₂O | xDMSO | xH₂O | mTotal |
| --- | --- | --- | --- | --- | --- |
|  | kmol | kmol | mol frac. | mol frac. | kg |
| 1 | 51.96 | 0.0255 | 0.9995 | 0.00049 | 4060.02 |
| 2 | 66.36 | 0.0323 | 0.9995 | 0.00049 | 5185.60 |
| 3 | 69.66 | 0.0331 | 0.9995 | 0.00048 | 5443.17 |
| 4 | 71.38 | 0.0343 | 0.9995 | 0.00048 | 5578.05 |
| 5 | 70.54 | 0.0330 | 0.9995 | 0.00047 | 5512.53 |
| 6 | 70.77 | 0.0328 | 0.9995 | 0.00046 | 5530.48 |

The DMSO bottom purity remained high, with an average value of 0.9995 mol fraction. However, the total DMSO bottom mass increased significantly compared with Method 1. This is expected because the water/DMSO run-off is recycled directly into the pot charge, causing more DMSO to remain within the main batch sequence.

## Comparison of the Two Recycling Methods

Comparison between method 1 and method 2.

| Parameter | Method 1 | Method 2 |
| --- | --- | --- |
| Avg. xIPA Dist. [mol frac.] | 0.9987 | 0.9987 |
| Total mDist. [kg] | 14775.70 | 14755.93 |
| Total t [h] | 111.00 | 120.30 |
| Total Bottom m [kg] | 25050.56 | 31309.85 |
| RO handled [kg] | 7669.81 H₂O/DMSO RO | 11110.08 combined RO |
| Recycle batches studied | 5 | 5 |
| Operating adjustments | Fewer | More |

Both recycling strategies achieved the required IPA product purity. The average IPA distillate purity was practically the same for both methods, at approximately 0.9987 mol fraction. However, Method 1 produced a slightly larger total IPA distillate mass and required less total operating time. Method 2 required 120.30 h compared with 111.00 h for Method 1, which means that total run-off re-integration increased the total process time by 9.30 h over the six simulated batches.

Method 2 also required more changes to the simulation settings to maintain convergence and product quality. This suggests that adding both run-off streams directly to the next pot charge makes the process more sensitive to composition changes. The larger DMSO bottom mass in Method 2 also shows that DMSO accumulates more strongly in the batch sequence when the water/DMSO stream is directly recycled.

Total run-off re-integration is technically possible, since it still achieves the required IPA purity and produces a high-purity DMSO bottom stream, and it actually allows us to recover more total DMSO (moles) from the bottom of the column. However, it is less favorable than Method 1 because it increases the operating time, requires more reflux ratio adjustments, and causes greater DMSO accumulation in the system; furthermore, even though more DMSO is recovered overall, each batch produces less quality bottom compared to Method 1. Based on this comparison, the preferred material recovery strategy is Method 1: water/IPA run-off re-integration combined with separate later treatment of the water/DMSO run-off. This approach gives similar product purity, slightly higher IPA recovery, shorter operating time, and better process stability. Since the reboiler’s heat duty is directly correlated to the elapsed time due to it being set at 1000 MJ/h, less time needed to complete the process means that it is more environmentally and financially affordable, not only less time consuming.

## Water/DMSO Run-off Distillation

The water/DMSO run-off distillation serves as a following step to the water/IPA re-integration. The same column is to be used for this process. The goal of this extra step is to recover as much DMSO and water as possible, for it to be repurposed later. The obtained, total amount of water moles and DMSO moles from the water/DMSO run-off were calculated after the 5 batches for Method 1, and this result was the pot charge for the distillation. There will be no feed or external entrainer. We created a separate ChemCAD simulation with the same column parameters and initial conditions.

Different stop values for the process were investigated, finally falling at 0.998 mol frac. water in the accumulator (water is more volatile than DMSO), as well as reflux values and internal pot pressure values. The reflux ratio was varied in the range R = 2 to R = 10, and the pressure was studied between 1 bar and 0.1 bar (Budapest University of Technology and Economics, Department of Building Service Engineering and Process Engineering 2023).

After many trials, it was determined that a high reflux ratio paired with vacuum conditions will be needed. This is due to the fact that, as the last traces of water begin to evaporate, at this temperature DMSO starts turning into vapour as well, rapidly, since the reboiler keeps providing heat. To obtain a purity of more than 99.75 mol% for both water and DMSO, the pressure must be taken down to 0.1 bar to stimulate quick water evaporation and make the gap between water’s and DMSO’s volatility bigger. In the same way, it was determined that a reflux ratio of 9 will be needed. As the cold reflux liquid flows downward, it meets the hot, rising DMSO vapor. This forces DMSO condense back into liquid, while water continues to vaporize and rise. The reason for the need behind such a high R value is time. Just as with pressure, the more time it takes to complete the operation, the more DMSO gets heated up and evaporated. With lower reflux ratios, it was not possible to find a middle ground where both the distillate and the bottoms are of acceptable quality.

However, the ChemCAD simulation needed additional aid in order to converge. Hold-up values were added: 5 dm3 for the condenser and 3 dm3 for the stages (Kiss 2013). These represent the liquid that gets “held up” and does not continue to form part of the distillate or does not fully evaporate, due to the geometry of the condenser and the type of packing. Adding hold up values also makes the process more accurate and realistic.

In order to maintain realism, several batches for both recycling methods were performed again, with the same hold-up values for the column as in the water/DMSO distillation. Our products (composition, mass, time) only slightly changed, affecting the values only after 2-3 decimal points. Purity of the distillate products remained acceptable. It was then concluded that it is not necessary to add hold-up values in a more complex ChemCAD simulation for accurate results.

The recovered product is summarized below:

- Mass of recovered water: 268.72 kg

- Purity of recovered water: 0.9975 mole fraction water

- Mass of recovered DMSO: 7401.16 kg

- Purity of recovered DMSO: 0.9975 mole fraction DMSO

- Total elapsed time: 6.55 h

As it can be seen, with Method 1 the yields are still better than Method 2, and the new elapsed time is still inferior (119.55 h compared to 122.3 h). Therefore, even though it includes more steps, it can be concluded that Method 1 is the most efficient.

Since this distillation process is under vacuum, a proper vacuum pump was selected for this application. A breakdown of the selection is carried out in the next sub-chapter.

## Vacuum Pump Selection

A vacuum pump was required to evacuate the distillation system prior to the start of each batch. The objective was to reduce the system pressure from atmospheric pressure (1.0 bar absolute) to the operating pressure of 0.1 bar absolute before heating commenced. Since the vacuum pump is only used during the initial evacuation stage and not during normal distillation operation, the pump was sized based on the required pump-down time and the total gas volume contained within the equipment.

The total volume to be evacuated consists of the vapor space in the feed tank and the internal volume of the distillation column. The feed tank has a total volume of 7.85 m³ and contains approximately 6 m³ of liquid before the start of the distillation, resulting in a vapor space of 1.85 m³. The distillation column has a height of 10 m and a diameter of 0.60 m, corresponding to an internal volume of approximately 2.83 m³. To allow for additional piping and system volume, the total gas volume to be evacuated was approximated as 5.0 m³.

The required pumping speed was calculated using the standard vacuum pump-down equation (Pfeiffer Vacuum, n.d.):

S=VtlnP1P2  (43)

where:

- S = required pumping speed (m³/s)

- V = gas volume to be evacuated (m³)

- t = desired pump-down time (s)

- P1 = initial pressure (bar abs)

- P2 = final pressure (bar abs)

Substituting the design values:

V=5.0 m3

We want the pump-down time to be approximately 20 minutes:

t=20×60=1200 s

This gives:

S=5.01200ln1.00.1=0.00959 m3/s=34.54 m3/h

Therefore, the vacuum system must provide a minimum pumping speed of approximately 35 m³/h to achieve the required vacuum level under the desired time.

A liquid ring vacuum pump was selected due to its suitability for handling condensable vapors and traces of process fluids. Compared to oil-sealed rotary vane pumps, liquid ring pumps are more tolerant to water vapor and solvent carryover, making them well suited for vacuum distillation applications. Since the process involves water and dimethyl sulfoxide (DMSO), a liquid ring design provides increased operational robustness and reliability (Busch Vacuum Solutions 2024).

The selected vacuum pump is the Busch DOLPHIN VX 0055, a total recirculation liquid ring vacuum pump. The manufacturer specifies a nominal pumping speed of 47 m³/h at 50 Hz and an ultimate pressure of 33 mbar absolute. Since the required operating pressure is 100 mbar absolute and the required pumping speed is 34.54 m³/h, the selected pump satisfies both requirements.

# Cost Estimation

The cost estimation of the distillation system was carried out using preliminary equipment cost correlations. This type of estimation is suitable for conceptual design because it allows the main process equipment costs to be estimated from the principal sizing parameters, such as heat-transfer area, column diameter, column height, tank dimensions, and compressor horsepower. The objective of this chapter is not to obtain a final vendor quotation, but to estimate the relative investment required for the main distillation system and for the two heat recycling alternatives. However, the costs of water will be neglected in this estimation.

The equipment cost correlations were taken from Douglas, where purchased equipment costs are estimated from equipment size and then corrected to installed costs using installation factors. Since the original correlations are based on an older cost index, the Marshall and Swift index correction was applied. The Marshall & Swift index used in this calculation, for the year 2020, was (Mondal and Jana 2022):

MS=2171.6

Two tanks were assumed: one tank for the pot charge and one tank for the distillate.

## Energy Cost estimation

The energy cost was estimated from the reboiler duty and the additional energy required to heat the DMSO feed from 20°C to 62°C. As of 2013 in the United States, literature reports a general electricity cost of approximately (Kiss 2013):

Cenergy=16.8 $/GJ

This value was used as the unit energy cost for the preliminary estimation. The energy cost was calculated by multiplying the total energy demand by this unit cost:

C=Qtotal⋅Cenergy  (44)

For the main IPA distillation step, the reboiler operated for 16.15 h at 1000 MJ/h:

Qreb,main=1000⋅16.15=16150 MJ=16.15 GJ  (45)

The DMSO feed-heating duty was approximately 50 MJ/h, this value was obtained from simulation measurements. Since the feed is heated during the main distillation step:

Qfeed=50⋅9.05=452.5 MJ=0.4525 GJ

Therefore, the total energy required for the main IPA distillation step was:

Qmain,total=16.15+0.4525=16.6025 GJ

Cmain=16.6025⋅16.8=278.92 $

## Heat Exchanger Cost Estimation

The heat-transfer areas used for the cost estimation were obtained from the previously calculated exchanger duties and temperature differences.

The resulting areas were converted from square meters to square feet using:

1 m2=10.764 ft2

The corrected heat-transfer areas used in the cost estimation are shown in the table below, their values converted from meters to feet, in order to follow the calculations from the selected literature.

Corrected heat-transfer areas used for cost estimation.

| Equipment | Area [ft²] |
| --- | --- |
| Condenser | 129.93 |
| Reboiler | 84.04 |
| Inter-cooler | 5.39 |
| Pre-heater | 6.65 |

The purchased cost of a heat exchanger was calculated as (Douglas 1988):

Cp,HX=MS280101.3A0.65Fc  (46)

where:

- Cp,HX = purchased heat exchanger cost, in $

- A = heat-transfer area, in ft²

- Fc = correction factor

- MS = Marshall and Swift index

The installed cost was calculated using (Douglas 1988):

Ci,HX=MS280101.3A0.65(Fc+2.29)  (47)

Since high-pressure steam was used as the heating utility for the reboiler, its cost must also be accounted for. The cost of high-pressure steam in terms of the heat exchanger heat duty is (Kiss 2013):

CHPS=9.88 $/GJ

Therefore, the total cost of high-pressure steam per batch was estimated from the reboiler heat duty and the operating time as follows:

Csteam=Qreb⋅t⋅CHPS=Csteam=1.00×16.15×9.88=159.56 $/batch

The total capital needed to install and operate the reboiler is:

Ci,reb,corr=Ci,reb+Csteam

Ci,reb,corr=54463.99+159.56=54623.55 $

The cost results for all heat exchangers used are shown below.

Heat exchanger cost estimation results.

| Equipment | Area [ft²] | Fc | Purchased [$] | Installed [$] |
| --- | --- | --- | --- | --- |
| Condenser | 129.93 | 0.80 | 14867.20 | 57424.56 |
| Reboiler | 84.04 | 1.60 | 22401.64 | 54623.55 |
| Inter-cooler | 5.39 | 0.80 | 1879.36 | 7259.03 |
| Pre-heater | 6.65 | 0.80 | 2154.16 | 8320.45 |

The condenser and reboiler are part of the base distillation system. The inter-cooler is only required for the heat pump method, while the pre-heater is only required for the pre-heater heat recycling method. The installed reboiler cost includes the estimated high-pressure steam cost for one complete batch.

## Tank Cost Estimation

Two tanks were included in the cost estimation: one for the pot charge and one for the distillate. The tank dimensions used in the calculation are shown in the table below.

Tank sizing values.

| Parameter | Symbol | Value | Unit |
| --- | --- | --- | --- |
| Tank height | H | 8.20 | ft |
| Tank diameter | D | 6.56 | ft |
| Number of tanks | N | 2 | - |
| Correction factor | Fc | 1.00 | - |

The purchased cost of the two tanks was calculated with (Douglas 1988):

Cp,tank=2MS280101.9D1.066H0.802Fc  (48)

Substituting the selected tank dimensions:

Cp,tank=63463.52$

The installed cost of the two tanks was calculated as well (Douglas 1988):

Ci,tank=2MS280101.9D1.066H0.802(Fc+2.18)  (49)

Substituting the selected values:

Ci,tank=201814.01$

## Column Cost Estimation

The cost of the distillation column was estimated using the column height and diameter. The column dimensions used in the cost estimation are shown in the following table, converted to feet:

Column sizing values.

| Parameter | Symbol | Value | Unit |
| --- | --- | --- | --- |
| Column height | H | 32.80 | ft |
| Column diameter | D | 2.49 | ft |
| Correction factor | Fc | 1.05 | - |

The purchased cost of the column shell was calculated using (Douglas 1988):

Cp,col=MS280101.9D1.066H0.802Fc=Cp,col=36217.64$  (50)

The installed cost of the column shell was calculated to be (Douglas 1988):

Ci,col=MS280101.9D1.066H0.802(Fc+2.49)=111412.36$  (51)

## Compressor Cost Estimation

The compressor costs correspond to the heat pump method. Two compressors are required, one for each compression stage. The compressor powers were obtained from the heat pump simulation and converted into horsepower, as shown in the following table:

Compressor power values.

| Equipment | Power [hp] |
| --- | --- |
| Compressor 1 | 36.36 |
| Compressor 2 | 37.15 |

The purchased compressor cost was calculated using the formula (Douglas 1988):

Cp,comp=MS28051.75HP0.82Ft  (52)

Since two compressors are required:

Cp,comp,total=MS28051.75HP10.82Ft+MS28051.75HP20.82Ft

where:

- HP1 = horsepower of compressor 1

- HP2 = horsepower of compressor 2

- Ft = compressor correction factor

Using Ft=1, the total purchased compressor cost is:

Cp,comp,total=15420.85$

The installed compressor cost was calculated using:

Ci,comp,total=MS28051.75HP10.82(Ft+2.11)+MS28051.75HP20.82(Ft+2.11)  (53)

Resulting in a cost for the heat pump method of 47958.84 $.

## Packing Cost Estimation

The selected column internal is MellapakPlus 252.Y structured packing. Since no vendor quotation was available, the packing cost was estimated from the random-packing cost data given by literature and then corrected with a structured-packing multiplier.

Stainless steel Pall rings are listed at approximately 1750−3200 $/m3 (Kiss 2013). Since the selected packing is metallic, the average value was used as the reference random-packing cost (after EUR to USD conversion, as of June 2026):

Crandom,unit=2475 $/m3

The column has a selected diameter of 0.60 m and a packed height of 10 m, giving a packing volume of:

Vpacking=2.827 m3

Therefore, the estimated equivalent random-packing cost is:

Crandom=2.827×2475=6997.90 $

Structured packing is reported to cost approximately 30−60% more per cubic meter than comparable metal random packing (SuTong Technology 2026). Therefore, the midpoint (45%) was used. The estimated structured-packing cost is therefore:

Cpacking=6997.90×1.45=10146.95 $  (54)

## Feed Cost Estimation

This subchapter estimates the cost of purchasing DMSO feed and compares it with the energy cost required to recover DMSO from the water/DMSO run-off stream. Since DMSO is the extractive solvent, recovering it reduces the amount of fresh solvent that must be purchased.

For the DMSO purchase price, a recent European market value was used (IMARC Group 2026). The price of DMSO in Germany was reported as, after converting the price from EUR to USD as of June 2026:

CDMSO=5.322 $/kg

From the material recovery chapter, the water/DMSO run-off distillation recovered 7401.16 kg of usable DMSO. The estimated cost of purchasing an equivalent amount of fresh DMSO would be:

CDMSO,purchase=7401.16×5.322=39388.97 $  (55)

The water/DMSO recovery distillation required a total elapsed time of 6.55 h. Since the reboiler duty was set at $1000 MJ/h, the total energy required for this recovery step was:

Erecovery=1000×6.55=6550 MJ=6.55 GJ

Using the general energy cost (Kiss 2013):

Cenergy=16.8 $/GJ

the energy cost of the DMSO recovery step is:

Crecovery=6.55×16.8=110.04 $

Comparison of DMSO purchase cost and recovery energy cost.

| Parameter | Value |
| --- | --- |
| DMSO purchase cost [$] | 39388.97 |
| Energy cost [$/GJ] | 16.8 |
| Recovery energy cost [$] | 110.04 |
| Net saving by recovery [$] | 39278.93 |

The comparison shows that recovering DMSO is much cheaper than purchasing the same amount of fresh solvent. The energy cost required to recover 7401.16 kg of DMSO is approximately 110$, while purchasing the same amount of DMSO would cost approximately 39388.97$. Therefore, the estimated net saving is 39278.93$. From a feed-cost perspective, DMSO recovery is financially advisable. In addition, it reduces solvent waste and improves the sustainability of the material recovery process.

## Base Distillation System Cost

The base distillation system includes the two tanks, the column shell, the column internals, condenser, and reboiler. The column internals correspond to the selected structured packing, with an estimated packing cost of:

Cpacking=10146.95 $

Base distillation system installed cost.

| Equipment | Installed Cost [$] |
| --- | --- |
| Two tanks | 201814.01 |
| Column shell | 111412.36 |
| Packing | 10146.95 |
| Condenser | 57424.56 |
| Reboiler | 54463.99 |

The total installed cost of the base system is:

Cbase=201814.01+111412.36+10146.95+57424.56+54463.99=435261.87$  (56)

## Cost of the Pre-heater Method

The pre-heater method only requires one additional heat exchanger; its installed cost was calculated earlier in this chapter:

Ci,pre=8320.45$

Therefore, the total installed cost of the system with the pre-heater method is 443582.32$.

## Cost of the Heat Pump Method

The heat pump method requires two compressors and an inter-cooler. The compressor cost was estimated earlier in this chapter using the compressor cost correlation (Douglas 1988). The total purchased compressor cost is:

Cp,comp,total=15420.85$

The installed cost of the two compressors is:

Ci,comp,total=47958.84$

The installed cost of the inter-cooler is:

Ci,int=7259.03$

Therefore, the total additional installed cost of the heat pump system is 55217.87$, making the grand total 490479.74$

## Comparison of Heat Recycling Alternatives

The two heat recycling alternatives were compared by considering their additional installed costs and their long-term energy savings. The pre-heater method requires only one additional heat exchanger, while the heat pump method requires two compressors and an inter-cooler.

Cost comparison of heat recycling methods.

| Method | Additional Equipment | Add. Installed Cost [$] |
| --- | --- | --- |
| Pre-heater | heat exch. | 8320.45 |
| Heat pump | 2 compressors + inter-cooler | 55217.87 |

From a capital cost perspective, the pre-heater method is the cheaper heat-recycling option. Its additional installed cost is 8320.45$, while the heat pump method requires 55217.87$, which is approximately 6.64 times higher. However, the long-term energy savings must also be considered; assuming one distillation cycle per day and a life expectancy of 10 years:

Ncycles=365×10=3650 cycles

Using the energy cost:

Cenergy=16.8 $/GJ

the estimated 10-year savings are shown below.

Ten-year comparison of heat recycling methods.

| Method | Energy Saved [GJ/batch] | Saving [$/batch] | 10-year Saving [$] | Add. Installed Cost [$] | Net 10-year Saving [$] | Payback [years] |
| --- | --- | --- | --- | --- | --- | --- |
| Pre-heater method | 0.408 | 6.86 | 25044.90 | 8320.45 | 16724.45 | 3.32 |
| Heat pump method | 3.504 | 58.86 | 214844.00 | 55217.87 | 159626.13 | 2.57 |

The pre-heater method saves less energy, but it also requires a much smaller investment. Over 10 years, it gives an estimated net saving of approximately 16724.45$, meaning that the installation cost is recovered and the method is financially justified. The heat pump method has a much higher installation cost, but it also saves much more energy. Over the same 10-year period, it gives an estimated net saving of approximately 159626.13$.

# Summary

This paper studied the regeneration of a pharmaceutical waste solvent by batch extractive distillation. The objective was to recover an IPA-rich distillate with at least 99.75 mol% IPA from an IPA–water mixture using DMSO as an extractive solvent. Since IPA and water form a minimum-boiling azeotrope, conventional batch distillation cannot reach the required purity. ChemCAD simulations were used to study the IPA–water–DMSO system and evaluate the effects of feed stage, reflux ratio, feed rate, number of stages, and feed temperature. Under the selected operating conditions, the main distillation step reached a maximum IPA purity of 99.856 mol% and lasted 9.05 hours.

Additional distillation stages were considered to recover remaining IPA, water, and DMSO from the bottom mixture, improving material recovery and reducing waste. Two recycling strategies were compared. The preferred option was the reintegration of the water/IPA run-off into the following batch, while collecting the water/DMSO run-off separately. This method provided similar product purity, slightly higher IPA recovery, shorter operating time, and better process stability.

The total energy requirement of the complete process was approximately 16.6025 GJ, corresponding to an estimated cost of 278.92$. Two heat-recycling alternatives were also studied. The pre-heater method was simpler and required much lower investment, while the heat pump method involved more equipment but offered greater energy savings, with an estimated net saving of 159626.13$ after 10 years.

The main process equipment was preliminarily sized, including the packed column, tanks, condenser, reboiler, heat-recovery equipment, and vacuum pump. The selected tank geometry had a diameter of 2.00 m, a height of 2.50 m, and a total volume of 7.85 m³. The estimated installed cost of the base distillation system was 435261.87$.

Overall, the results show that batch extractive distillation with DMSO is a feasible method for regenerating high-purity IPA from pharmaceutical waste solvent streams. The proposed process supports solvent recovery, run-off reuse, waste reduction, and improved energy efficiency, making it a technically viable and potentially sustainable regeneration solution.

ASME. 2025a. BPVC Section II: Materials, Part d: Properties (Metric). American Society of Mechanical Engineers.

ASME. 2025b. BPVC Section VIII: Rules for Construction of Pressure Vessels, Division 1. American Society of Mechanical Engineers.

Budapest University of Technology and Economics, Department of Building Service Engineering and Process Engineering. 2023. Lecture Notes v: Distillation.

Budapest University of Technology and Economics, Department of Building Service Engineering and Process Engineering. 2024a. Heat Exchangers 1.

Budapest University of Technology and Economics, Department of Building Service Engineering and Process Engineering. 2024b. Heat Exchangers 2: Determination of Heat Transfer Surface Area of a Tubular Heat Exchanger.

Busch Vacuum Solutions. 2024. DOLPHIN VX 0030/0055 A Total Recirculation. https://www.buschvacuum.com/hu/en/products/dolphin-vx-0030-0055-a-total-recirculation.html.

Cengel, Y. A., and A. J. Ghajar. 2011. Heat and Mass Transfer: Fundamentals and Applications. 4th ed. McGraw-Hill.

Coulson, J. M., J. F. Richardson, J. R. Backhurst, and J. H. Harker. 1999. Coulson & Richardson’s Chemical Engineering, Volume i: Shell and Tube Heat Exchangers.

Douglas, J. M. 1988. Conceptual Design of Chemical Processes. McGraw-Hill.

Fonyó, Zsolt, and György Fábry. 2004. Vegyipari művelettani Alapismeretek. Nemzeti Tankönyvkiadó.

Gerbaud, V., I. Rodriguez-Donis, L. Hegely, P. Lang, F. Denes, and X. You. 2019. “Review of Extractive Distillation: Process Design, Operation, Optimization and Control.” Chemical Engineering Research and Design 141: 229–71.

Hilburger, M. W. 2020. Buckling of Thin-Walled Circular Cylinders. NASA/SP-20205011530. National Aeronautics; Space Administration.

IMARC Group. 2026. Dimethyl Sulfoxide (DMSO) Price Trend, Index and Forecast. https://www.imarcgroup.com/dimethyl-sulfoxide-pricing-report.

Kemp, I. C. 2007. Pinch Analysis and Process Integration: A User Guide on Process Integration for the Efficient Use of Energy. 2nd ed. Butterworth-Heinemann.

Kern, D. Q. 1950. Process Heat Transfer. McGraw-Hill.

Kiss, A. A. 2013. Advanced Distillation Technologies: Design, Control and Applications. John Wiley & Sons.

Mondal, B., and A. K. Jana. 2022. “Optimal Reflux Splitting Reactive Distillation for Algal Biodiesel Production: Waste Heat Recovery Through Vapor Recompression and Organic Rankine Cycle.” Separation and Purification Technology 292.

Nemeth, B., L. Hegely, and P. Lang. 2019. “Comparison of Batch Heteroazeotropic Distillation Operational Strategies for the Dehydration of Isopropanol.” Chemical Engineering Research and Design 146.

Perry, R. H., and D. W. Green, eds. 2008. Perry’s Chemical Engineers’ Handbook. 8th ed. McGraw-Hill.

Pfeiffer Vacuum. n.d. Vacuum Calculations: Basic Formulas. Pfeiffer Vacuum. https://www.pfeiffervacuum.com/tr/en/knowledge/vacuum-technology/knowledge-book/2-basic-calculations/2.2-calculations.html.

Seader, J. D., E. J. Henley, and D. K. Roper. 2011. Separation Process Principles: Chemical and Biochemical Operations. 3rd ed. John Wiley & Sons.

Sulzer Chemtech. 2024. Structured Packings: Energy-Efficient, Innovative & Profitable. Sulzer.

SuTong Technology. 2026. Random Vs Structured Packing: Performance, Cost, and Selection Guide. https://www.sutongtechnology.com/blog/random-vs-structured-packing-comparison.html.

Yao, J. Y., S. Y. Ling, and I. L. Chien. 2007. “Operation and Control of Batch Extractive Distillation for the Separation of Mixtures with Minimum Boiling Azeotrope.” Journal of the Chinese Institute of Chemical Engineers 38: 371–83.